MasterSpec 2014

SECTION 01 74 19

CONSTRUCTION WASTE MANAGEMENT AND DISPOSAL

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

GENERAL

RELATED DOCUMENTS

MANDATORY REQUIREMENT:  Retain this article in all Sections of Project Manual.

Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 01 Specification Sections, apply to this Section.

SUMMARY

Section includes administrative and procedural requirements for the following:

Salvaging nonhazardous demolition and construction waste.

Recycling nonhazardous demolition and construction waste.

Disposing of nonhazardous demolition and construction waste.

Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

Section 02 41 16 "Structure Demolition" for disposition of waste resulting from demolition of buildings, structures, and site improvements.

Section 02 41 19 "Selective Demolition" for disposition of waste resulting from partial demolition of buildings, structures, and site improvements.

Section 31 10 00 "Site Clearing" for disposition of waste resulting from site clearing and removal of above - and below-grade improvements.

DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

Source Separated Recycling Facility (SSRF): A facility that exclusively accepts separated individual commodities for the purpose of recycling; such as metals, paper, wood, and/or inerts such as asphalt and concrete.

Mixed Debris: Includes solid items such as building materials, packaging, and rubble resulting from construction, remodeling, repair, and demolition operations.  One mixed debris processing facility is located in San Diego County at EDCO, 6670 Federal Blvd, Lemon Grove, CA 91945, herein referred to as the EDCO Mixed Debris Recycling Facility.

Class III Landfill: A landfill that accepts non-hazardous waste such as household, commercial, and industrial waste.

Administrative Recycling Program: Separation and recovery of paper and beverage containers from both permanent administrative offices and construction site   office(s).

Construction Waste: Building and site improvement materials and other solid waste resulting from construction, remodeling, renovation, or repair operations. Construction waste includes packaging.

Demolition Waste: Building and site improvement materials resulting from demolition or selective demolition operations.

Disposal: Removal off-site of demolition and construction waste and subsequent sale, recycling, reuse, or deposit in landfill or incinerator acceptable to authorities having jurisdiction.

Recycle: Recovery of demolition or construction waste for subsequent processing in preparation for reuse.

Salvage: Recovery of demolition or construction waste and subsequent sale or reuse in another facility.

Salvage and Reuse: Recovery of demolition or construction waste and subsequent incorporation into the Work.

PERFORMANCE REQUIREMENTS

General: Achieve end-of-Project rates for salvage/recycling of 75 percent by weight of total non-hazardous solid waste generated by the Work. Practice efficient waste management in the use of materials in the course of the Work. Use all reasonable means to divert construction and demolition waste from landfills and incinerators.  Clearly label all recycling containers and list acceptable and unacceptable materials. Deliver recyclable materials to source separated recycling facilities.  Facilitate recycling and salvage of materials, including the following as applicable:

Demolition Waste:

Asphalt paving.

Concrete.

Concrete reinforcing steel.

Brick.

Concrete masonry units.

Wood studs.

Wood joists.

Plywood and oriented strand board.

Wood paneling.

Wood trim.

Structural and miscellaneous steel.

Rough hardware.

Roofing.

Insulation.

Doors and frames.

Door hardware.

Windows.

Glazing.

Metal studs.

Gypsum board.

Acoustical tile and panels.

Carpet.

Carpet pad.

Demountable partitions.

Equipment.

Cabinets.

Plumbing fixtures.

Piping.

Supports and hangers.

Valves.

Sprinklers.

Mechanical equipment.

Refrigerants.

Electrical conduit.

Copper wiring.

Lighting fixtures.

Lamps.

Ballasts.

Electrical devices.

Switchgear and panelboards.

Transformers.

Construction Waste:

Masonry and CMU.

Lumber.

Wood sheet materials.

Wood trim.

Metals.

Roofing.

Insulation.

Carpet and pad.

Gypsum board.

Piping.

Electrical conduit.

Packaging: Regardless of salvage/recycle goal indicated in "General" Paragraph above, salvage or recycle 100 percent of the following uncontaminated packaging materials:

Paper.

Cardboard.

Boxes.

Plastic sheet and film.

Polystyrene packaging.

Wood crates.

Plastic pails.

Co-mingled Debris: Direct all co-mingled site tonnage to the EDCO Mixed Debris Processing Facility.

ACTION SUBMITTALS

Waste Management Plan: Submit plan within 10 days of date established for the Notice to Proceed.

INFORMATIONAL SUBMITTALS

SDUSD Contractor Summary Site Debris Diversion Report:  Concurrent with each Application for Payment, submit report. Use District Form CSDDR-1, attached at the end of this Section.  The District Construction Manager will provide an editable version.  Failure to include Report will result in a 10 percent withholding of payment.

Waste Reduction Calculations: Before request for Substantial Completion, submit calculated end-of-Project rates for salvage, recycling, and disposal as a percentage of total waste generated by the Work.

Records of Donations: Indicate receipt and acceptance of salvageable waste donated to individuals and organizations.

Records of Sales: Indicate receipt and acceptance of salvageable waste sold to individuals and organizations.

Recycling and Processing Facility Records: Indicate receipt and acceptance of recyclable waste by recycling and processing facilities licensed to accept them. Include manifests, weight tickets, receipts, and invoices.

Landfill and Incinerator Disposal Records: Indicate receipt and acceptance of waste by landfills and incinerator facilities licensed to accept them. Include manifests, weight tickets, receipts, and invoices.

Coordinate "Qualification Data" Paragraph below with "Quality Assurance" Article.

Qualification Data: For Waste Management Coordinator.

QUALITY ASSURANCE

Regulatory Requirements: Comply with hauling and disposal regulations of authorities having jurisdiction.

Waste Management Conference: Prior to commencement of work, conduct conference at Project site.  Attendees shall include District Construction Manager, Waste Management Coordinator, and Contractor personnel involved in demolition and waste handling.  Review methods and procedures related to waste management, including:

Retain subparagraphs below if required. Revise to suit Project.

Review and discuss waste management plan including responsibilities of Waste Management Coordinator.

Review requirements for documenting quantities of each type of waste and its disposition.

Review and finalize procedures for materials separation and verify availability of containers and bins needed to avoid delays.

Review procedures for periodic waste collection and transportation to recycling and disposal facilities.

Review waste management requirements for each trade.

SITE DEBRIS MANAGEMENT PLAN

General: Develop a site debris management plan.  Use District Form CSDMP-1, attached at the end of this Section.  The District Construction Manager will provide an editable version.  Use a separate form for each project phase (land clearing, demolition, construction).

Post approved plan in a prominent location at the Project site and distribute copies to superintendent and all subcontractors.

PRODUCTS (Not Used)

EXECUTION

PLAN IMPLEMENTATION

General: Implement approved waste management plan. Provide handling, containers, storage, signage, transportation, and other items as required to implement waste management plan during the entire duration of the Contract.

Edit "Waste Management Coordinator" Paragraph below to suit size and complexity of Project.

Waste Management Coordinator: Engage a waste management coordinator to be responsible for implementing, monitoring, and reporting status of waste management work plan. Coordinator shall be present at the Project site full time during land clearing and site demolition phases, and part time as needed during construction phase

Training: Train workers, subcontractors, and suppliers on proper waste management procedures, as appropriate for the Work.

Distribute waste management plan to everyone concerned within three days of submittal return.

Distribute waste management plan to entities when they first begin work on-site. Review plan procedures and locations established for salvage, recycling, and disposal.

Site Access and Temporary Controls: Conduct waste management operations to ensure minimum interference with roads, streets, walks, walkways, and other adjacent occupied and used facilities.

Designate and label specific areas on Project site necessary for separating materials that are to be salvaged, recycled, reused, donated, and sold.

END OF SECTION 01 74 19

Retain this notice in all Sections of Project Manual.

San Diego Unified School District Guide Specifications

Document Version: August 2021

# FORM CSDMP-1

SDUSD CONTRACTOR SITE DEBRIS MANAGEMENT PLAN (CSDMP)

| Complete a separate form for each project phase (i.e. demolition, land clearing, construction) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Title: |  |  |  |  |  |  |  |  |  |  |
| Contract or Work Order No.: |  |  |  |  |  |  |  |  |  |  |
| Contractor's Name: |  |  |  |  |  |  |  |  |  |  |
| Street Address: |  |  |  |  |  |  |  |  |  |  |
| City: |  |  |  |  | State: |  |  |  | Zip: |  |
| Phone: ( ) |  |  |  |  | Fax: ( ) |  |  |  |  |  |
| E-Mail Address: |  |  |  |  |  |  |  |  |  |  |
| Prepared by: (Print Name) |  |  |  |  |  |  |  |  |  |  |
| Date Submitted: |  |  |  |  |  |  |  |  |  |  |
| Reuse, Recycling or Mixed Debris Processing Processes Used |  |  |  |  |  |  |  |  |  |  |
| Describe the types of recycling processes or disposal activities used for material generated in the project. Indicate the type of process or activity by number, types of materials, and quantities that are estimated for reuse and recycling below: |  |  |  |  |  |  |  |  |  |  |
| 01 - Reuse of building materials or salvage items on site (i.e. fencing or red clay brick) |  |  |  |  |  |  |  |  |  |  |
| 02 - Salvaging building materials or salvage items at an offsite salvage or re-use center (i.e. lighting, fixtures) |  |  |  |  |  |  |  |  |  |  |
| 03 - Recycling source separated materials on site (i.e. crushing asphalt/concrete for reuse or grinding for mulch) |  |  |  |  |  |  |  |  |  |  |
| 04 - Recycling source separated materials at an offsite recycling center (i.e. scrap metal or green matls) |  |  |  |  |  |  |  |  |  |  |
| 05 - Recycling commingled loads of C&D matls at EDCO Mixed Debris Recycling Facility |  |  |  |  |  |  |  |  |  |  |
| 06 - Recycling material as Alternative Daily Cover at landfills |  |  |  |  |  |  |  |  |  |  |
| 07 - Delivery of soils or mixed inerts to an inert landfill for disposal (inert fill). |  |  |  |  |  |  |  |  |  |  |
| 09 - Other (please describe) _______________________________________________________________ |  |  |  |  |  |  |  |  |  |  |
| Types of Material Generated |  |  |  |  |  |  |  |  |  |  |
| Use these codes to indicate the types of material that are estimated to be generated on the project |  |  |  |  |  |  |  |  |  |  |
| A = Asphalt |  | C = Concrete |  | - M = Metals |  | I = Mixed Inert |  |  | G = Green Matls |  |
| D = Drywall |  | P/C=Paper/Cardboard |  | W/C = Wire/Cable |  | S= Soils (Non Hazardous) |  |  |  |  |
| M/C = Miscellaneous Construction Debris |  |  |  | R = Reuse/Salvage |  | W = Wood |  |  | O = Other (describe) |  |
| Facilities Used: Provide Name of Facility and Location (City) |  |  |  |  |  |  |  |  |  |  |
| Total Truck Loads: Provide Number of Trucks Hauled from Site During Reporting Period |  |  |  |  |  |  |  |  |  |  |
| Total Quantities: If scales are available at sites, report in tons. If not, quantify by cubic yards. For salvage/reuse items, quantify by estimated weight (or units). Provide weight slips or load tickets for each load delivered. |  |  |  |  |  |  |  |  |  |  |
| SECTION I - RE-USED/RECYCLED MATERIALS |  |  |  |  |  |  |  |  |  |  |
| Include all proposed recycling activities for source separated recycling centers. |  |  |  |  |  |  |  |  |  |  |
| Type of Material | Type of Activity |  | Facilities Used/ Location |  |  | Total Truck Loads | Total Quantities |  |  |  |
|  |  |  |  |  |  |  | Tons | Cubic YD |  | Other Wt. |
| (ex.) M | 04 |  | ABC Metals, National City |  |  | 24 | 355 |  |  |  |
| a. Source Separated Diversion |  |  |  |  |  | 0 | 0 | 0 |  | 0 |

| FORM CSDMP-1 (Continued) SDUSD CONTRACTOR SITE DEBRIS MANAGEMENT PLAN (CSDMP) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SECTION II - MIXED DEBRIS PROCESSING MATERIALS |  |  |  |  |  |  |  |  |
| Include estimates of all debris generated from activities where no source separated recycling will occur. |  |  |  |  |  |  |  |  |
| Type of Material | Type of Activity |  | Facilities Used/ Location |  | Total Truck Loads | - Total Quantities |  |  |
|  |  |  |  |  |  | Tons | Cubic YD | Other Wt. |
| (ex.) M/C | 5 |  | EDCO Mixed Debris Recycling Facility |  | 2 | 35 |  |  |
| SECTION III - TOTAL MATERIALS GENERATED EDCO |  |  |  |  |  |  |  |  |
| This section calculates the total materials recycled verses the total materials disposed for mixed debris sent to EDCO |  |  |  |  |  |  |  |  |
|  |  | Tons Recycled (tons x 0.80) |  | Tons Disposed (tons x 0.20) |  |  |  |  |
| a. EDCO |  |  |  |  |  |  |  |  |
| SECTION IV - CONTRACTOR'S LANDFILL DIVERSION RATE CALCULATION |  |  |  |  |  |  |  |  |
| Add totals from Section I + Section II +Section III |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Tons | Cubic Yards | Other Wt. |
| a. Materials Re-Used and Recycled (Section I + II +III) |  |  |  |  |  |  |  |  |
| - b. EDCO Disposal (Section III) |  |  |  |  |  |  |  |  |
| c. Total Materials Generated (a. + b. = c.) |  |  |  |  |  |  |  |  |
| d. Landfill Diversion Rate (a/c = d Tons Only)* |  |  |  |  |  |  |  |  |
| * Use tons only to calculate recycling percentages: Tons Reused/Recycled/Tons Generated = % Recycled |  |  |  |  |  |  |  |  |
| Contractor's Comments (Provide any additional information pertinent to planned reuse, recycling, or disposal activities): |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |  |
| - EDCO will recover 80% of the mixed debris for the purposes of recycling. Therefore, multiply tonnage by 0.80 for tons recycled and multiply tonnage by 0.20 for total project disposal. - Suggested Conversion Factors: From Cubic Yards to Tons |  |  |  |  |  |  |  |  |
| Asphalt: 0.61 (ex. 1000 CY Asphalt = 610 tons. Applies to broken chunks of asphalt) |  |  |  |  |  |  |  |  |
| Concrete: 0.93 (ex. 1000 CY Concrete = 930 tons. Applies to broken chunks of concrete) |  |  |  |  |  |  |  |  |
| Ferrous Metals: 0.22 (ex. 1000 CY Ferrous Metal = 220 tons) |  |  |  |  |  |  |  |  |
| Non-Ferrous Metals: 0.10 (ex. 1000 CY Non-Ferrous Metals = 100 tons) |  |  |  |  |  |  |  |  |
| Drywall Scrap: 0.20 |  |  |  |  |  |  |  |  |
| Wood Scrap: 0.16 |  |  |  |  |  |  |  |  |

Section B: Plan Narrative -- Methods to Ensure Diversion

Describe the method to be used to reuse and recycle (methods shall include one or more of the following:  deconstruction to salvage all or most materials generated, selective salvage with source separation, and/or reuse of materials onsite):

Describe methods to be used to provide onsite instruction regarding appropriate separation, handling, recycling, salvage, reuse and return methods to achieve waste reduction goals.

Describe methods to be used to protect materials to be recycled from contamination. Including schedule of regular clean-up, schedule visual inspections of dumpsters and recycling bins to identify potential contamination of materials.

How will materials be stored and how much space will be required?

Describe your administrative recycling program.

FORM CSDDR-1

SDUSD CONTRACTOR SUMMARY SITE DEBRIS DIVERSION REPORT (CSDDR)

| (Submit With Each Progress Payment) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Title: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contract or Work Order No.: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor's Name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Street Address: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| City: |  |  |  |  |  |  |  | State: |  |  |  |  | Zip: |  |  |
| Phone: ( ) |  |  |  |  |  |  |  | Fax: ( ) |  |  |  |  |  |  |  |
| E-Mail Address: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Prepared by: (Print Name) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date Submitted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Period Covered: |  |  |  |  | From: |  |  |  |  |  | To: |  |  |  |  |
| Reuse, Recycling or Mixed Debris Processing Processes Used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Describe the types of recycling processes or disposal activities used for material generated in the project. Indicate the type of process or activity by number, types of materials, and quantities that were recycled or disposed in the sections below: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 01 - Reuse of building materials or salvage items on site (i.e. fencing or red clay brick) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 02 - Salvaging building materials or salvage items at an offsite salvage or re-use center (i.e. lighting, fixtures) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 03 - Recycling source separated materials on site (i.e. crushing asphalt/concrete for reuse or grinding for mulch) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 04 - Recycling source separated materials at an offsite recycling center (i.e. scrap metal or green matls) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 05 - Recycling commingled loads of C&D matls at EDCO Mixed Debris Recycling Facility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 06 - Recycling material as Alternative Daily Cover at landfills |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 07 - Delivery of soils or mixed inerts to an inert landfill for disposal (inert fill). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 09 - Other (please describe) _______________________________________________________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Types of Material Generated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use these codes to indicate the types of material that were generated on the project |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A = Asphalt |  |  | C = Concrete |  |  | M = Metals |  | I = Mixed Inert |  |  |  |  | G = Green Matls |  |  |
| D = Drywall |  |  | P/C=Paper/Cardboard |  |  | W/C = Wire/Cable |  | S= Soils (Non Hazardous) |  |  |  |  |  |  |  |
| M/C = Miscellaneous Construction Debris |  |  |  |  |  | R = Reuse/Salvage | W = Wood |  |  |  |  |  | O = Other (describe) |  |  |
| Facilities Used: Provide Name of Facility and Location (City) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Truck Loads: Provide Number of Trucks Hauled from Site During Reporting Period |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Quantities: If scales are available at sites, report in tons. If not, quantify by cubic yards. For salvage/reuse items, quantify by estimated weight (or units). Provide weight slips or load tickets for each load delivered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION I - RE-USED/RECYCLED MATERIALS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Include all recycling activities for source separated recycling centers where recycling occurred. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of Material | Type of Activity |  |  | Facilities Used/ Location |  |  |  |  | Total Truck Loads | Total Quantities |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Tons |  | Cubic YD |  |  | Other Wt. |
| (ex.) M | 04 |  |  | ABC Metals, National City |  |  |  |  | 24 | 355 |  |  |  |  |  |
| a. Source Separated Diversion |  |  |  |  |  |  |  |  | 0 | 0 |  | 0 |  |  | 0 |

| FORM CSDDR-1 (Continued) SDUSD CONTRACTOR SUMMARY SITE DEBRIS DIVERSION REPORT (CSDDR) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SECTION II - MIXED DEBRIS PROCESSING MATERIALS |  |  |  |  |  |  |  |
| Include all debris generating activities for materials that were not send to source separated recycling facilities. |  |  |  |  |  |  |  |
| Type of Material | Type of Activity | Facilities Used/Location |  | Total Truck Loads | Total Quantities |  |  |
|  |  |  |  |  | Tons | Cubic YD | Other Wt. |
| (ex.) M/C | 05 | EDCO Mixed Debris Recycling Facility |  | 2 | 35 |  |  |
| SECTION III - TOTAL MATERIALS GENERATED EDCO |  |  |  |  |  |  |  |
| This section calculates the total materials recycled verses the total materials disposed for mixed debris sent to EDCO |  |  |  |  |  |  |  |
|  |  | Tons Recycled (tons x 0.80) | Tons Disposed (tons x 0.20) |  |  |  |  |
| a. EDCO |  |  |  |  |  |  |  |
| SECTION IV - CONTRACTOR'S LANDFILL DIVERSION RATE CALCULATION |  |  |  |  |  |  |  |
| Add totals from Section I + Section II |  |  |  |  |  |  |  |
|  |  |  |  |  | Tons | Cubic YD | Other Wt. |
| a. Materials Re-Used and Recycled (Section I + II +III) |  |  |  |  | 0 |  |  |
| b. EDCO Disposal (Section III) |  |  |  |  | 0 |  |  |
| c. Total Materials Generated (a. + b. = c.) |  |  |  |  | 0 |  |  |
| d. Landfill Diversion Rate (a/c = d Tons Only)* |  |  |  |  |  |  |  |
| * Use tons only to calculate recycling percentages: Tons Reused/Recycled/Tons Generated = % Recycled |  |  |  |  |  |  |  |
| Contractor's Comments (Provide any additional information pertinent to planned reuse, recycling, or disposal activities): |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |
| - EDCO will recover 80% of the mixed debris for the purposes of recycling. Therefore, multiply tonnage by 0.80 for tons recycled and multiply tonnage by 0.20 for total project disposal. - Suggested Conversion Factors: From Cubic Yards to Tons (Use when scales are not available) |  |  |  |  |  |  |  |
| Asphalt: 0.61 (ex. 1000 CY Asphalt = 610 tons. Applies to broken chunks of asphalt) |  |  |  |  |  |  |  |
| Concrete: 0.93 (ex. 1000 CY Concrete = 930 tons. Applies to broken chunks of concrete) |  |  |  |  |  |  |  |
| Ferrous Metals: 0.22 (ex. 1000 CY Ferrous Metal = 220 tons) |  |  |  |  |  |  |  |
| Non-Ferrous Metals: 0.10 (ex. 1000 CY Non-Ferrous Metals = 100 tons) |  |  |  |  |  |  |  |
| Drywall Scrap: 0.20 |  |  |  |  |  |  |  |
| Wood Scrap: 0.16 |  |  |  |  |  |  |  |