MasterSpec 2014

SECTION 07 21 00

THERMAL INSULATION

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

GENERAL

RELATED DOCUMENTS

MANDATORY REQUIREMENT:  Retain this article in all Sections of Project Manual.

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

SUMMARY

Section Includes:

Glass-fiber blanket.

Glass-fiber board.

Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

Section 07 54 19 "Polyvinyl-Chloride (PVC) Roofing" for insulation specified as part of roofing construction.

ACTION SUBMITTALS

Product Data: For each type of product.

INFORMATIONAL SUBMITTALS

Product Test Reports: For each product, for tests performed by a qualified testing agency.

DELIVERY, STORAGE, AND HANDLING

Protect insulation materials from physical damage and from deterioration due to moisture, soiling, and other sources. Store inside and in a dry location. Comply with manufacturer's written instructions for handling, storing, and protecting during installation.

PRODUCTS

GLASS-FIBER BLANKET

Copy paragraphs below and re-edit for each type of insulation required. Insert drawing designations to identify each product. Use these designations on Drawings to show where each insulation type is required.

Glass-Fiber Blanket, Unfaced: ASTM C 665, Type I; with maximum flame-spread and smoke-developed indexes of 25 and 50, respectively, per ASTM E 84; passing ASTM E 136 for combustion characteristics.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

CertainTeed Corporation.

Johns Manville; a Berkshire Hathaway company.

Owens Corning.

Or Equal.

Glass-Fiber Blanket, Reinforced-Foil Faced: ASTM C 665, Type III (reflective faced), Class A (faced surface with a flame-spread index of 25 or less); Category 1 (membrane is a vapor barrier), faced with foil scrim, foil-scrim kraft, or foil-scrim polyethylene.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

CertainTeed Corporation.

Johns Manville; a Berkshire Hathaway company.

Owens Corning.

Or Equal.

GLASS-FIBER BOARD

Copy paragraphs below and re-edit for each type of insulation required. Insert drawing designations to identify each product. Use these designations on Drawings to show where each insulation type is required.

Glass-Fiber Board, Unfaced: ASTM C 612, Type IA; unfaced, with maximum flamespread and smoke-developed indexes of 25 and 50, respectively, per ASTM E 84, passing ASTM E 136 for combustion characteristics. Nominal density of 2.25 lb/cu. ft, thermal resistivity of 4.3 deg F x h x sq. ft./Btu x in. at 75 deg F.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

CertainTeed Corporation.

Johns Manville; a Berkshire Hathaway company.

Owens Corning.

Or Equal.

Glass-Fiber Board, Faced: ASTM C 612, Type IA; faced on one side with foil-scrim-kraft or foil-scrim-polyethylene vapor retarder, with maximum flame-spread and smoke-developed indexes of 25 and 50, respectively, per ASTM E 84. Nominal density of 2.25 lb/cu. ft., thermal resistivity of 4.3 deg F x h x sq. ft./Btu x in. at 75 deg F.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

CertainTeed Corporation.

Johns Manville; a Berkshire Hathaway company.

Owens Corning.

Or Equal.

INSULATION FASTENERS

Anchor in "Adhesively Attached, Spindle-Type Anchors" Paragraph below is an example of a mechanical fastener. Prong anchors, welding pins, pointed rods, for example, are also available; insert here as required.

Adhesively Attached, Spindle-Type Anchors: Plate welded to projecting spindle; capable of holding insulation of specified thickness securely in position with self-locking washer in place.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

AGM Industries, Inc.

Cal-Fasteners, Inc.

Integrity Fasteners, Inc.

Or Equal.

Revise "Plate" and "Spindle" subparagraphs below if stainless steel or another metal is required.

Plate: Perforated, galvanized carbon-steel sheet, 0.030-inch thick by 2 inches square.

Spindle: Copper-coated, low-carbon steel; fully annealed; 0.105 inch in diameter; length to suit depth of insulation.

Revise size in "Insulation-Retaining Washers" Paragraph below if required.

Insulation-Retaining Washers: Self-locking washers formed from 0.016-inch-thick galvanized-steel sheet, with beveled edge for increased stiffness, sized as required to hold insulation securely in place, but not less than 1-1/2 inches square or in diameter.

Manufacturers: Subject to compliance with requirements, provide products by the following:

AGM Industries, Inc.

Cal-Fasteners, Inc.

Integrity Fasteners, Inc.

Or Equal.

Retain subparagraph below if anchors are used in crawl spaces, ceiling plenums, attic spaces, and so forth, where sharp ends of spindles would be exposed to human contact.

Protect ends with capped self-locking washers incorporating a spring steel insert to ensure permanent retention of cap in the following locations:

Crawl spaces.

Ceiling plenums.

Attic spaces.

Anchor Adhesive: Product with demonstrated capability to bond insulation anchors securely to substrates without damaging insulation, fasteners, or substrates.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

AGM Industries, Inc.

Cal-Fasteners, Inc.

Gemco.

Or Equal.

ACCESSORIES

Retain "Insulation for Miscellaneous Voids" Paragraph below for miscellaneous voids if needed for thermal protection or air-infiltration reduction.

Insulation for Miscellaneous Voids:

Glass-Fiber Insulation: ASTM C 764, Type II, loose fill; with maximum flame-spread and smoke-developed indexes of 5, per ASTM E 84.

Retain "Adhesive for Bonding Insulation" Paragraph below for insulation adhesively bonded to substrates.

Adhesive for Bonding Insulation: Product compatible with insulation and air and water barrier materials, and with demonstrated capability to bond insulation securely to substrates without damaging insulation and substrates.

EXECUTION

PREPARATION

Clean substrates of substances that are harmful to insulation, including removing projections capable of puncturing insulation or vapor retarders, or that interfere with insulation attachment.

INSTALLATION, GENERAL

Comply with insulation manufacturer's written instructions applicable to products and applications.

Install insulation that is undamaged, dry, and unsoiled and that has not been left exposed to ice, rain, or snow at any time.

Extend insulation to envelop entire area to be insulated. Fit tightly around obstructions and fill voids with insulation. Remove projections that interfere with placement.

Provide sizes to fit applications and selected from manufacturer's standard thicknesses, widths, and lengths. Apply single layer of insulation units unless multiple layers are otherwise shown or required to make up total thickness or to achieve R-value.

INSTALLATION OF INSULATION IN FRAMED CONSTRUCTION

Blanket Insulation: Install in cavities formed by framing members according to the following requirements:

Use insulation widths and lengths that fill the cavities formed by framing members. If more than one length is required to fill the cavities, provide lengths that will produce a snug fit between ends.

Place insulation in cavities formed by framing members to produce a friction fit between edges of insulation and adjoining framing members.

Maintain 3-inch clearance of insulation around recessed lighting fixtures not rated for or protected from contact with insulation.

Retain first subparagraph below for eave ventilation.

Attics: Install eave ventilation troughs between roof framing members in insulated attic spaces at vented eaves.

Retain first subparagraph below for metal-framed construction.

For metal-framed wall cavities where cavity heights exceed 96 inches, support unfaced blankets mechanically and support faced blankets by taping flanges of insulation to flanges of metal studs.

Retain first subparagraph below for wood-framed construction.

For wood-framed construction, install blankets according to ASTM C 1320 and as follows:

With faced blankets having stapling flanges, lap blanket flange over flange of adjacent blanket to maintain continuity of vapor retarder once finish material is installed over it.

Vapor-Retarder-Faced Blankets: Tape joints and ruptures in vapor-retarder facings, and seal each continuous area of insulation to ensure airtight installation.

Specify location of vapor retarder in "Exterior Walls" Subparagraph below based on vapor-flow analysis of a construction assembly.

Exterior Walls: Set units with facing placed toward interior of construction, unless otherwise indicated on Drawings.

Retain "Interior Walls" Subparagraph below for interior walls around high-humidity areas such as shower rooms and swimming pools.

Interior Walls: Set units with facing placed as indicated on Drawings and toward areas of high humidity.

Miscellaneous Voids: Install insulation in miscellaneous voids and cavity spaces where required to prevent gaps in insulation using the following materials:

Glass-Fiber Insulation: Compact to approximately 40 percent of normal maximum volume equaling a density of approximately 2.5 lb/cu. ft.

PROTECTION

Protect installed insulation from damage due to harmful weather exposures, physical abuse, and other causes. Provide temporary coverings or enclosures where insulation is subject to abuse and cannot be concealed and protected by permanent construction immediately after installation.

END OF SECTION 07 21 00

Retain this notice in all Sections of Project Manual.

San Diego Unified School District Guide Specifications

Document Version: August 2021