MasterSpec 2012

SECTION 07 31 13

ASPHALT SHINGLES

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

GENERAL

RELATED DOCUMENTS

MANDATORY REQUIREMENT:  Retain this article in all Sections of Project Manual.

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

SUMMARY

Section Includes:

Asphalt shingles.

Underlayment.

Ridge vents.

Metal flashing and trim.

DEFINITION

Roofing Terminology: See ASTM D 1079 and glossary of NRCA's "The NRCA Roofing and Waterproofing Manual" for definitions of terms related to roofing work in this Section.

PREINSTALLATION MEETINGS

Always retain "Preinstallation Conference" Paragraph below.  Revise to suit Project.

Preinstallation Conference: Conduct conference at Project site.

Meet with District Construction Manager, Project Inspector, Architect, testing and inspecting agency representative, roofing Installer, roofing system manufacturer's representative, deck Installer, and installers whose work interfaces with or affects roofing, including installers of roof accessories and roof-mounted equipment.

Review methods and procedures related to roofing installation, including manufacturer's written instructions.

Review and finalize construction schedule, and verify availability of materials, Installer's personnel, equipment, and facilities needed to make progress and avoid delays.

Examine deck substrate conditions and finishes for compliance with requirements, including flatness and fastening.

Review structural loading limitations of roof deck during and after roofing.

Review special roofing details, roof drainage, roof penetrations, equipment curbs, and condition of other construction that affects roofing system.

Review governing regulations.

Review temporary protection requirements for roofing system during and after installation.

Review roof observation and repair procedures after roofing installation.

ACTION SUBMITTALS

Product Data: For each type of product.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

Samples: For each exposed product and for each color and texture specified.

Asphalt Shingles: Full size.

Ridge and Hip Cap Shingles: Full size.

Ridge Vent: 12-inch-long Sample.

Exposed Valley Lining: 12 inches square.

Samples for Initial Selection: For each type of asphalt shingle indicated.

Include similar Samples of accessories involving color selection.

Samples for Verification: For the following products, of sizes indicated:

Asphalt Shingles: Full size.

Ridge and Hip Cap Shingles: Full size.

Ridge Vent: 12-inch-long Sample.

Exposed Valley Lining: 12 inches square.

INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in "Quality Assurance" Article.

Qualification Data: For Installer.

Product Test Reports: For each type of asphalt shingle and underlayment product indicated, for tests performed by a qualified testing agency.

Sample Warranty: For manufacturer's warranty.

CLOSEOUT SUBMITTALS

Maintenance Data: For asphalt shingles to include in maintenance manuals.

Information Card:  Furnish a typewritten card, laminated in plastic.  Card shall be 8 1/2-by-11 inches and shall contain the information listed on Form 1 located at end of this section.

QUALITY ASSURANCE

Retain "Installer Qualifications" Paragraph below if required for workmanship component of manufacturer's special material-and-workmanship warranty.

Installer Qualifications: An authorized representative who is trained and approved by manufacturer.

DELIVERY, STORAGE, AND HANDLING

Store roofing materials in a dry, well-ventilated location protected from weather, sunlight, and moisture according to manufacturer's written instructions.

Store underlayment rolls on end on pallets or other raised surfaces. Do not double stack rolls.

Protect unused roofing materials from weather, sunlight, and moisture when left overnight or when roofing work is not in progress.

Handle, store, and place roofing materials in a manner to prevent damage to roof deck or structural supporting members.

WARRANTY

Manufacturer's Warranty: Manufacturer agrees to repair or replace asphalt shingles that fail within specified warranty period.

Failures include:

Manufacturing defects.

Material Warranty Period: Minimum 30 years from date of Substantial Completion, prorated, with first five years nonprorated.

Wind-Speed Warranty Period: Asphalt shingles will resist blow-off or damage caused by wind speeds of up to 90 mph for minimum five years from date of Substantial Completion.

Roofing Installer's Warranty: Installer agrees to repair or replace components of asphalt-shingle roofing that fail in materials or workmanship within specified warranty period.

Warranty Period: Two years from date of Substantial Completion.

PRODUCTS

PERFORMANCE REQUIREMENTS

Retain this article for Class A roof assemblies. Glass-fiber-reinforced asphalt shingles complying with ASTM D 3462/D 3462M have a Class A rating according to ASTM E 108 or UL 790. Verify that underlayment products comply with requirements.

Exterior Fire-Test Exposure: Provide asphalt shingles and related roofing materials identical to those of assemblies tested for Class A fire resistance according to ASTM E 108 or UL 790 by Underwriters Laboratories or another testing and inspecting agency acceptable to authorities having jurisdiction. Identify products with appropriate markings of applicable testing agency.

Retain "Energy Performance" Paragraph below for roofs that must comply with the CBC. Options are values required for steep-slope roofs by prescriptive approach; revise if other values are required for building-envelope trade-off approach or whole-building performance approach. A list of products tested according to CRRC-1 with their test values is available in PDF at www.coolroofs.org.

Energy Performance: Roofing system shall have a minimum aged solar reflectance of 0.20 and a minimum thermal emittance of 0.75, or a minimum SRI of 16, when tested according to CRRC-1.

GLASS-FIBER-REINFORCED ASPHALT SHINGLES

Laminated-Strip Asphalt Shingles: ASTM D 3462/D 3462M, laminated, multi-ply overlay construction, glass-fiber reinforced, mineral-granule surfaced, and self-sealing.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

CertainTeed Corporation.

GAF Materials Corporation.

Owens Corning.

Or Equal.

Butt Edge: Straight cut.

Strip Size: Manufacturer's standard.

Color and Blends: As indicated by manufacturer's designations.

Three-Tab-Strip Asphalt Shingles: ASTM D 3462/D 3462M, glass-fiber reinforced, mineral-granule surfaced, and self-sealing; with tabs regularly spaced.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

CertainTeed Roofing Corporation.

GAF Materials Corporation.

Owens Corning.

Or Equal.

Strip Size: Manufacturer's standard.

Color and Blends: As indicated by manufacturer's designations.

Retain one of two options in "Hip and Ridge Shingles" Paragraph below based on type of shingle selected and roof configuration.

Hip and Ridge Shingles: Manufacturer's standard units to match asphalt shingles.

UNDERLAYMENT MATERIALS

Felt: ASTM D 226/D 226M, asphalt-saturated organic felts, nonperforated.

Type: Type I.

Retain "Granular-Surfaced Valley Lining" Paragraph below if required.

Granular-Surfaced Valley Lining: ASTM D 3909, mineral-granular-surfaced, glass-felt-based, asphalt roll roofing; 36 inches wide.

RIDGE VENTS

Retain this article if ridge venting is required.

Rigid Ridge Vent: Manufacturer's standard, rigid section high-density polypropylene or other UV-stabilized plastic ridge vent for use under ridge shingles.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

Air Vent, Inc.; a Gibraltar Industries company.

Cor-A-Vent, Inc.

Owens Corning.

Or Equal.

Flexible Ridge Vent: Manufacturer's standard, compression-resisting, three-dimensional, open-nylon or polyester-mat filter.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

Air Vent, Inc.; a Gibraltar Industries company.

Cor-A-Vent, Inc.

Owens Corning.

Or Equal.

ACCESSORIES

Asphalt Roofing Cement: ASTM D 4586, Type II, asbestos free.

Roofing Nails: ASTM F 1667; aluminum, stainless-steel, copper, or hot-dip galvanized-steel wire shingle nails, minimum 0.120-inch-diameter, sharp-pointed, with a minimum 3/8-inch-diameter flat head and of sufficient length to penetrate 3/4-inch into solid wood decking or extend at least 1/8-inch through plywood sheathing.

Shank: Barbed.

Where nails are in contact with metal flashing, use nails made from same metal as flashing.

Felt-Underlayment Nails: Aluminum, stainless-steel, or hot-dip galvanized-steel wire with low-profile capped heads or disc caps, 1-inch minimum diameter.

METAL FLASHING AND TRIM

General: Comply with requirements in Section 07 62 00 "Sheet Metal Flashing and Trim."

Sheet Metal: As indicated.

Fabricate sheet metal flashing and trim to comply with recommendations in SMACNA's "Architectural Sheet Metal Manual" that apply to design, dimensions, metal, and other characteristics of the item.

Retain "Apron Flashings," "Step Flashings," "Cricket or Backer Flashings," "Open-Valley Flashings," and "Drip Edges" subparagraphs below if Drawings do not fully detail these flashing conditions. Review ARMA recommendations or those in NRCA's "The NRCA Roofing Manual: Steep-Slope Roof Systems"; revise to suit Project.

Retain "Vent Pipe Flashings" Paragraph below for plumbing vents that penetrate roof.

Vent Pipe Flashings: Provide roof-penetration flashing soldered to skirt at slope of roof, and extending at least 4 inches from pipe onto roof.  Provide with vandal resistant pipe cap.

EXECUTION

EXAMINATION

Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

Examine roof sheathing to verify that sheathing joints are supported by framing and blocking or metal clips and that installation is within flatness tolerances.

Verify that substrate is sound, dry, smooth, clean, sloped for drainage, and completely anchored; and that provisions have been made for flashings and penetrations through asphalt shingles.

Proceed with installation only after unsatisfactory conditions have been corrected.

UNDERLAYMENT INSTALLATION

General: Comply with underlayment manufacturer's written installation instructions applicable to products and applications indicated unless more stringent requirements apply.

Retain "Single-Layer Felt Underlayment" Paragraph below if required. CBC requires roofs with a slope of 4:12 or greater to be installed with a single underlayment layer.

Single-Layer Felt Underlayment: Install on roof deck parallel with and starting at the eaves. Lap sides a minimum of 2 inches over underlying course. Lap ends a minimum of 4 inches. Stagger end laps between succeeding courses at least 72 inches. Fasten with felt underlayment and roofing nails.

Retain "Double-Layer Felt Underlayment" Paragraph below if required. CBC requires roofs with slopes of 2:12, but less than 4:12, to be installed with a second underlayment layer. Side lap and starter course dimensions are based on 36-inch- (914-mm-) wide rolls of felt underlayment and are needed to maintain double-layer felt underlayment coverage with 2 inches (50-mm) to spare.

Double-Layer Felt Underlayment: Install on roof deck parallel with and starting at the eaves. Install a 19-inch-wide starter course at eaves and completely cover with full-width second course. Install succeeding courses lapping previous courses 19 inches in shingle fashion. Lap ends a minimum of 6 inches. Stagger end laps between succeeding courses at least 72 inches. Fasten with felt underlayment and roofing nails.

Terminate felt underlayment extended up not less than 4 inches against sidewalls, curbs, chimneys, and other roof projections.

Retain" Concealed Valley Lining" Paragraph below for lining closed-cut valleys with granular-surfaced valley lining.

Concealed Valley Lining: For closed-cut valleys. Comply with NRCA's "NRCA Guidelines for Asphalt Shingle Roof Systems." Install underlayment centered in valley and fastened to roof deck.

Lap roof-deck underlayment over valley underlayment at least 6 inches.

Install a 36-inch-wide strip of granular-surfaced valley lining, with granular-surface face up, centered in valley. Lap ends of strips at least 12 inches in direction to shed water, and seal with asphalt roofing cement. Fasten to roof deck.

Retain "Metal-Flashed, Open-Valley Underlayment" Paragraph below for underlayment at open valleys to be covered with metal valley flashing.

Metal-Flashed, Open-Valley Underlayment: Install two layers of minimum 36-inch-wide underlayment centered in valley. Stagger end laps between layers at least 72 inches. Lap ends of each layer at least 12 inches in direction to shed water, and seal with asphalt roofing cement. Fasten each layer to roof deck.

Lap roof-deck underlayment over first layer of valley underlayment at least 6 inches.

Retain "Granular-Surfaced, Open-Valley Lining" Paragraph below for open valleys lined with exposed granular-surfaced valley lining.

Granular Surfaced, Open-Valley Lining: Comply with NRCA's "NRCA Guidelines for Asphalt Shingle Roof Systems." Install a 36-inch-wide felt underlayment centered in valley. Fasten to roof deck with felt underlayment and roofing nails.

Lap roof-deck felt underlayment over valley felt underlayment at least 6 inches.

Install an 18-inch-wide strip of valley lining centered in valley, with granular-surface face down. Install a second 36-inch-wide strip of valley lining centered in valley, with granular-surface face up. Lap ends of each strip at least 12 inches in direction to shed water, and seal with asphalt roofing cement. Stagger end laps between succeeding strips at least 72 inches. Fasten each strip to roof deck with roofing nails.

METAL FLASHING INSTALLATION

General: Install metal flashings and other sheet metal to comply with requirements in Section 07 62 00 "Sheet Metal Flashing and Trim."

Install metal flashings according to recommendations in NRCA's "NRCA Guidelines for Asphalt Shingle Roof Systems."

Retain "Apron Flashings" Paragraph below if required. Apron flashings provide a weatherproofing transition material where a roof area intersects a head wall. Common locations for apron flashings are the front downslope side of a dormer or chimney, curbed roof penetrations, and clerestory transitions.

Apron Flashings: Extend lower flange over and beyond each side of downslope asphalt shingles and up the vertical surface.

Retain "Step Flashings" Paragraph below for interleaving metal step flashings between succeeding courses of asphalt shingles that abut vertical surfaces, such as chimneys, sidewalls, and skylights.

Step Flashings: Install with a headlap of 2 inches and extend over the underlying asphalt shingle and up the vertical surface. Fasten to roof deck only.

Retain "Cricket or Backer Flashings" Paragraph below if required.

Cricket or Backer Flashings: Install against the roof-penetrating element extending concealed flange beneath upslope asphalt shingles and beyond each side.

Retain "Open-Valley Flashings" Paragraph below if metal open-valley flashings are required. Revise dimensions to suit Project.

Open-Valley Flashings: Install centered in valleys, lapping ends at least 8 inches in direction to shed water. Fasten upper end of each length to roof deck beneath overlap.

Retain first subparagraph below if securing flange edges with cleats.

Secure hemmed flange edges into metal cleats spaced 12 inches apart and fastened to roof deck.

Retain "Rake Drip Edges" and "Eave Drip Edges" paragraphs below if applicable.

Rake Drip Edges: Install rake drip-edge flashings over underlayment and fasten to roof deck.

Eave Drip Edges: Install eave drip-edge flashings below underlayment and fasten to roof sheathing.

Pipe Flashings: Form flashing around pipe penetrations and asphalt shingles. Fasten and seal to asphalt shingles as recommended by manufacturer.

ASPHALT-SHINGLE INSTALLATION

General: Install asphalt shingles according to manufacturer's written instructions and recommendations in NRCA's "NRCA Guidelines for Asphalt Shingle Roof Systems."

Install starter strip along lowest roof edge, consisting of an asphalt-shingle with self-sealing strip face up at roof edge.

Extend asphalt shingles 1/2-inch over fasciae at eaves and rakes.

Install starter strip along rake edge.

Retain first paragraph below for laminated-strip asphalt shingles. Diagonal installation is recommended by NRCA. Insert a particular offset pattern if required.

Install first and remaining courses of asphalt shingles stair-stepping diagonally across roof deck with manufacturer's recommended offset pattern at succeeding courses, maintaining uniform exposure.

Retain first paragraph below for three-tab-strip asphalt shingles. Diagonal installation is recommended by NRCA.

Install first and remaining courses of asphalt shingles stair-stepping diagonally across roof deck with manufacturer's recommended offset pattern at succeeding courses, maintaining uniform exposure.

Retain first paragraph below if using patterned shingles where manufacturer recommends racking method of installation.

Install asphalt shingles by single-strip column or racking method, maintaining uniform exposure. Install full-length first course followed by cut second course, repeating alternating pattern in succeeding courses.

Fasten asphalt-shingle strips with a minimum of four roofing nails located according to manufacturer's written instructions.

Retain first subparagraph below for roofs with slopes greater than 21:12.

Where roof slope exceeds 21:12, seal asphalt shingles with asphalt roofing cement spots after fastening with additional roofing nails.

Retain first subparagraph below for roofs with slopes less than 4:12.

Where roof slope is less than 4:12, seal asphalt shingles with asphalt roofing cement spots.

Retain "Closed-Cut Valleys" Paragraph below if required. Valley type depends on partly on type of shingle selected.

Closed-Cut Valleys: Extend asphalt-shingle strips from one side of valley [12 inches] <Insert dimension> beyond center of valley. Use one-piece shingle strips without joints in valley. Fasten with extra nail in upper end of shingle. Install asphalt-shingle courses from other side of valley and cut back to a straight line 2 inches short of valley centerline. Trim upper concealed corners of cut-back shingle strips.

Do not nail asphalt shingles within 6 inches of valley center.

Set trimmed, concealed-corner asphalt shingles in a 3-inch-wide bed of asphalt roofing cement.

Retain "Open Valleys" Paragraph below if required. Valley type depends partly on type of shingle selected.

Open Valleys: Cut and fit asphalt shingles at open valleys, trimming upper concealed corners of shingle strips. [Maintain uniform width of exposed open valley] [Widen exposed portion of open valley 1/8-inch in 12 inches] from highest to lowest point.

Set valley edge of asphalt shingles in a 3-inch-wide bed of asphalt roofing cement.

Do not nail asphalt shingles to metal open-valley flashings.

Ridge Vents: Install continuous ridge vents over asphalt shingles according to manufacturer's written instructions. Fasten with roofing nails of sufficient length to penetrate sheathing.

Hip and Ridge Shingles: Maintain same exposure of cap shingles as roofing shingle exposure. Lap cap shingles at ridges to shed water away from direction of prevailing winds. Fasten with roofing nails of sufficient length to penetrate sheathing.

Retain subparagraph below if retaining ridge vents.

Fasten ridge cap asphalt shingles to cover ridge vent without obstructing airflow.

END OF SECTION 07 31 13

Retain this notice in all Sections of Project Manual.

San Diego Unified School District Guide Specifications

Document Version: August 2021

1.	Location:___	______________________________	2.	Bldg. Name: 	_______________

3.	Bldg. No.:______	____4.	Roof Area (SF):	_______	5.	Contract No.:		____________

6.	Deck Slope:___________________

7.	Type of Deck:

( )	Metal								( )	Wood Plank or Plywood

( )	Cast-In-Place Concrete				( )	Other:_______________________

( )	Precast/Prestressed Concrete

8.	Type of Insulation Board:

( )	Polyisocyanurate/Composite		( )	Polyisocyanurate

( )	Polystyrene/Composite				( )	Polystyrene

( )	Perlite								( )	Mineral Fiber

( )	Other:_______________________________________

9.	Insulation Manufacturer:____________________________________________

10.	Insulation Thickness:_______________________________________________

11.	Roofing Type:

( )	Built-Up (Asphalt)					( )	PVC

( )	Metal								( )	SBS Mod. Bitumen

( )	Shingles							( )	Other: ____________

12.	Roofing Manufacturer:_____________________________________________

13.	Roofing Installer/Warrantor:_________________________________________

14.	Roofing Application Method:

( )	Bitumen							( )	Fully Adhered

( )	Mechanically Fastened 				( )	Mechanically Fastened/Fully Adhered

( )	Other:_________________________________________

15.	Warranty Period:	From:________________	To:___________

16.	Warranty Serial Number:____________________________________________

17.	Date Roofing Completed:_________18.	Inspector:________________________

19.	Prime Contractor Name/Address:______________________________________

Signature:__________________________________Date:_______________________

INSTRUCTIONS FOR FORM 1 (Do Not Post)

1.	Location:	Name of facility as shown on contract.

2.	Bldg. Name:	As shown on contract or as provided by District Construction Manager.

3.	Bldg. Number:	As provided by District Construction Manager.

4.	Roof Area:	Area in square feet of roof for which deck insulation, membrane, etc. are the same.  A separate form is required if any part of roof system is different over other areas of the roof.

5.	Contract Number:	As shown on the contract.

6.	Show deck slope.

7.	Deck:	Check appropriate block.

8.	Type of Insulation Board:	Check appropriate block.

9.	Show manufacturer of insulation.

10.	Show minimum thickness of installed insulation.

11.	Roofing Type:	Check appropriate block.

12.	Show roofing manufacturer's name.

13.	Roofing Installer's or Contractor's name.

14.	Roofing Application Method:	Check appropriate block.

15.	Warranty Period:	Insert start and end dates.

16.	Warranty Serial Number:	Insert serial number.

17.	Show date of Substantial Completion.	Warranty period begins on this date.

18.	Show Project Inspector's name.

19.	Prime Contractor Name/Address/Signature:	Must be signed and dated by an official of Contracting firm.