MasterSpec 2011

SECTION 31 50 00

EXCAVATION SUPPORT AND PROTECTION

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

GENERAL

RELATED DOCUMENTS

MANDATORY REQUIREMENT:  Retain this article in all Sections of Project Manual.

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

SUMMARY

Section includes temporary excavation support and protection systems.

Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

Section 01 32 33 "Photographic Documentation" for recording preexisting conditions and excavation support and protection system progress.

Section 01 57 23 “Temporary Storm Water Pollution Control” for excavation protection.

Section 31 20 00 "Earth Moving" for excavating and backfilling and for controlling surface-water runoff and ponding.

Section 31 23 19 "Dewatering" for dewatering excavations.

PRE-INSTALLATION MEETINGS

Retain "Pre-installation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

Pre-installation Conference: Conduct conference at Project site.

If needed, insert list of conference participants not mentioned in Section 01 31 00 "Project Management and Coordination."

Retain one or more subparagraphs below if additional agenda items specific to Project are required but not mentioned in Section 01 31 00 "Project Management and Coordination"; revise to suit Project.

Review geotechnical report.

Review existing utilities and subsurface conditions.

Review coordination for interruption, shutoff, capping, and continuation of utility services.

Review proposed excavations.

Review proposed equipment.

Review monitoring of excavation support and protection system.

Review coordination with waterproofing.

Review abandonment or removal of excavation support and protection system.

INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in "Quality Assurance” Article.

Qualification Data: For [land surveyor] [and] [professional engineer].

Contractor Calculations: For excavation support and protection system. Include analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

Usually retain "Existing Conditions" Paragraph below for determining the effect of excavation support and protection system on permanent construction.

Existing Conditions: Provide [photographs] [or] [video recordings] showing existing conditions of adjacent construction and site improvements in accordance with Section 01 32 33 “Photographic Documentation”.

Revise "Record Drawings" Paragraph below to suit Project. Coordinate with Section 01 77 00 "Closeout Procedures."

Record Drawings: Identify locations and depths of capped utilities, abandoned-in-place support and protection systems, and other subsurface structural, electrical, or mechanical conditions.

QUALITY ASSURANCE

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 01 40 02 "Quality Requirements / Contractor Laboratory."

Professional Engineer Qualifications:  A professional engineer who is legally qualified to practice in the State of California and who is experienced in providing engineering services for designing excavation support and protection systems that are similar to those indicated for this Project in material, design, and extent.

Engineering Responsibility:  Engage a qualified professional engineer to prepare or supervise the preparation of data for the excavation support and protection system including drawings and comprehensive engineering analysis that shows the system's compliance with specified requirements.

FIELD CONDITIONS

Retain "Interruption of Existing Utilities" Paragraph below if interruption of existing utility service is required.

Interruption of Existing Utilities: Do not interrupt any utility serving facilities occupied by District or others unless permitted under the following conditions and then only after arranging to provide temporary utility according to requirements indicated:

Notify District Construction Manager no fewer than [two] days in advance of proposed interruption of utility.

Do not proceed with interruption of utility without District Construction Manager's written permission.

Revise or delete "Project-Site Information" Paragraph below to suit Project and office practice.

Project-Site Information: A geotechnical report has been prepared for this Project and is available for information only. The opinions expressed in this report are those of a geotechnical engineer and represent interpretations of subsoil conditions, tests, and results of analyses conducted by a geotechnical engineer. District is not responsible for interpretations or conclusions drawn from the data.

Make additional test borings and conduct other exploratory operations necessary for excavation support and protection according to the performance requirements.

The geotechnical report is referenced elsewhere in Project Manual.

Survey Work: Engage a qualified land surveyor or professional engineer to survey adjacent existing buildings, structures, and site improvements; establish exact elevations at fixed points to act as benchmarks. Clearly identify benchmarks and record existing elevations.

During installation of excavation support and protection systems, regularly resurvey benchmarks, maintaining an accurate log of surveyed elevations for comparison with original elevations.  Promptly notify District Construction Manager if changes in elevations occur or if cracks, sags, or other damage is evident in adjacent construction.

PRODUCTS

PERFORMANCE REQUIREMENTS

Provide, design, monitor, and maintain excavation support and protection system capable of supporting excavation sidewalls and of resisting earth and hydrostatic pressures and superimposed and construction loads.

Contractor Design: Design excavation support and protection system, including comprehensive engineering analysis by a qualified professional engineer.

Prevent surface water from entering excavations by grading, dikes, or other means.

Install excavation support and protection systems without damaging existing buildings, structures, and site improvements adjacent to excavation.

Continuously monitor vibrations, settlements, and movements to ensure stability of excavations and constructed slopes and to ensure that damage to permanent structures is prevented.

EXECUTION

PREPARATION

Protect structures, utilities, sidewalks, pavements, and other facilities from damage caused by settlement, lateral movement, undermining, washout, and other hazards that could develop during excavation support and protection system operations.

Shore, support, and protect utilities encountered.

Install excavation support and protection systems to ensure minimum interference with roads, streets, walks, and other adjacent occupied and used facilities.

Do not close or obstruct streets, walks, or other adjacent occupied or used facilities without permission from District and authorities having jurisdiction. Provide alternate routes around closed or obstructed traffic ways if required by authorities having jurisdiction.

Locate excavation support and protection systems clear of permanent construction so that construction and finishing of other work is not impeded.

Promptly repair damages to adjacent facilities caused by installing excavation support and protection systems.

FIELD QUALITY CONTROL

Survey-Work Benchmarks: Resurvey benchmarks [daily] [weekly] during installation of excavation support and protection systems, excavation progress, and for as long as excavation remains open. Maintain an accurate log of surveyed elevations and positions for comparison with original elevations and positions. Promptly notify District Construction Manager if changes in elevations or positions occur or if cracks, sags, or other damage is evident in adjacent construction.

Promptly correct detected bulges, breakage, or other evidence of movement to ensure that excavation support and protection system remains stable.

Promptly repair damages to adjacent facilities caused by installation or faulty performance of excavation support and protection systems.

REMOVAL AND REPAIRS

Retain first paragraph below if removal of excavation support and protection systems is required.

Remove excavation support and protection systems when construction has progressed sufficiently to support excavation and earth and hydrostatic pressures. Remove in stages to avoid disturbing underlying soils and rock or damaging structures, pavements, facilities, and utilities.

Retain first subparagraph below if required.

Remove excavation support and protection systems to a minimum depth of 48 inches below overlying construction and abandon remainder.

Fill voids immediately with approved backfill compacted to density specified in Section 31 20 00 "Earth Moving."

Repair or replace, as approved by District Construction Manager, adjacent work damaged or displaced by removing excavation support and protection systems.

Retain paragraph below if excavation support and protection system will be left in place.

Leave excavation support and protection systems permanently in place.

END OF SECTION 31 50 00

Retain this notice in all Sections of Project Manual.

San Diego Unified School District Guide Specifications

Document Version: August 2021