MasterSpec 2014

SECTION 32 17 13

PARKING BUMPERS

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

GENERAL

RELATED DOCUMENTS

MANDATORY REQUIREMENT:  Retain this article in all Sections of Project Manual.

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

SUMMARY

Section includes wheel stops.

ACTION SUBMITTALS

Product Data: For each type of product.

PROJECT CONDITIONS

Traffic Control:  Maintain access for vehicular and pedestrian traffic as required for other construction activities.

PRODUCTS

PARKING BUMPERS

Revise "Concrete Wheel Stops" Paragraph below to suit Project.

Concrete Wheel Stops: Precast, steel-reinforced, air-entrained concrete, [4000-psi] minimum compressive strength, [4-1/2 inches high by 9 inches wide by 72 inches long]. Provide chamfered corners, transverse drainage slots on underside, and a minimum of [two] [three] factory-formed or -drilled vertical holes through wheel stop for anchoring to substrate.

Surface Appearance: Free of pockets, sand streaks, honeycombs, and other obvious defects. Corners shall be uniform, straight, and sharp.

In "Mounting Hardware" Subparagraph below, first option is generally used for asphalt surfaces and second is generally used for concrete. Consult manufacturer for recommendations.

Mounting Hardware:  At each wheel stop provide [two] [three] galvanized-steel [spikes or dowels, 3/4-inch diameter, 10-inch minimum length] [lag screws, shields, and washers; 1/2-inch diameter, 8-inch minimum length].

EXECUTION

EXAMINATION

Verify that pavement is in suitable condition to begin installation according to manufacturer's written instructions.

Proceed with installation only after unsatisfactory conditions have been corrected.

INSTALLATION

General: Install wheel stops according to manufacturer's written instructions unless otherwise indicated.

Install wheel stops in bed of adhesive before anchoring.

Adhesively bond anchors to both wheel stop and pavement.

At asphalt paving, extend upper portion of anchor 4-1/2-inches into wheel stop and a minimum of 5-1/2-inches into pavement.

Recess head of hardware beneath top of wheel stop.  Fill recess with sealant per Section 07 92 00 “Joint Sealants”.

END OF SECTION 32 17 13

Retain this notice in all Sections of Project Manual.

San Diego Unified School District Guide Specifications

Document Version: August 2021