MasterSpec 2014

SECTION 32 17 23

PAVEMENT MARKINGS

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

GENERAL

RELATED DOCUMENTS

MANDATORY REQUIREMENT:  Retain this article in all Sections of Project Manual.

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

SUMMARY

Section includes painted markings applied to [asphalt] [and] [concrete] pavement.

Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

Section 09 91 13 "Exterior Painting" for painting exterior concrete surfaces other than pavement.

PRE-INSTALLATION MEETINGS

Retain "Pre-installation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

Pre-installation Conference: Conduct conference at Project site.

If needed, insert list of conference participants not mentioned in Section 01 31 00 "Project Management and Coordination."

Retain subparagraph below if additional requirements are necessary. Insert additional requirements to suit Project.

Review methods and procedures related to marking pavement, including:

Pavement aging period before application of pavement markings.

Review requirements for protecting pavement markings, including restriction of traffic during installation period.

ACTION SUBMITTALS

Product Data:  For each type of product.

Include technical data and tested physical and performance properties.

Retain paragraph below if playground markings are included in the project.  Coordinate with “Playground Markings” Paragraph.

QUALITY ASSURANCE

Retain "Regulatory Requirements" Paragraph below if pavement-marking work is located on public property and must comply with requirements of state or local DOT.

Regulatory Requirements: Comply with materials, workmanship, and other applicable requirements of CBC Title 24 for pavement-marking work.

Building Code Accessibility Requirements:

1. Accessible parking spaces serving a particular building or facility shall be located

on the shortest accessible route to an entrance complying with CBC Section 11B-    208.3.1

Retain “Playground Markings” Paragraph below if existing playground markings will need to be replaced, or if new playground markings are to be provided.

[PLAYGROUND MARKINGS

Where existing playground markings will be disrupted by the Work of this project, survey, photograph and record all existing markings, and replace them in their original configuration, location and color.

Where new playground markings are to be provided, locate and configure as shown in the Drawings.]

DELIVERY, STORAGE AND HANDLING

Deliver pavement-marking materials to Project site in original packages with seals unbroken and bearing manufacturer’s labels containing brand name and type of material, date of manufacture, and directions for storage.

Store pavement-marking materials in a clean, dry, protected location within temperature range required by the manufacturer.  Protect stored materials from direct sunlight.

FIELD CONDITIONS

Environmental Limitations:  Proceed with pavement marking only on clean, dry surfaces and at a minimum ambient or surface temperature of [40 deg F for alkyd materials] [55 deg F for water-based materials], and not exceeding 95 deg F.

PRODUCTS

MANUFACTURERS

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

Dunn-Edwards Corporation.

Frazee Paint; Comex Group.

Sherwin-Williams Company (The).

Or Equal.

PAVEMENT-MARKING PAINT

Pavement-Marking Paint:  Alkyd traffic-marking paint.

Retain required color(s) in "Color" Subparagraph below. If more than one color is required, indicate locations of each on Drawings or by inserts.

Color: [White] [Yellow] [Blue] [Red] [As indicated] .

Pavement-Marking Paint: Latex, waterborne emulsion, lead and chromate free, ready mixed, complying with FS TT-P-1952E, Type II, with drying time of less than [three] [45] minutes.

Retain required color(s) in "Color" Subparagraph below. If more than one color is required, indicate locations of each on Drawings or by inserts.

Color: [White] [Yellow] [Blue] [Red] [As indicated].

Pavement-Marking Paint: Latex traffic-marking paint.

Retain required color(s) in "Color" Subparagraph below. If more than one color is required, indicate locations of each on Drawings or by inserts.

Color: [White] [Yellow] [Blue] [Red] [As indicated].

EXECUTION

EXAMINATION

Verify that pavement is dry and in suitable condition to begin pavement marking according to manufacturer's written instructions.

Proceed with pavement marking only after unsatisfactory conditions have been corrected.

PAVEMENT MARKING

Do not apply pavement-marking paint until layout, colors, and placement have been verified with the District Project Manager and District Construction Manager.

Marking-paint manufacturers caution that paint will bleed or tear surface of new asphalt unless asphalt is aged before painting. This aging period should be at least 30 days. If pavement marking must proceed immediately, consider revising text to a phased application of a thin first coat followed by a thicker second coat once asphalt has aged. Verify that two-coat application is recommended by marking-paint manufacturer.

Allow paving to age for a minimum of [30] days before starting pavement marking.

Sweep and clean surface to eliminate loose material and dust.

Retain option in paragraph below if retaining waterborne emulsion or latex pavement-marking paint.

Apply paint with mechanical equipment to produce pavement markings, of dimensions indicated, with uniform, straight edges. Apply at manufacturer's recommended rates to provide a minimum wet film thickness of [15 mils].

Retain first subparagraph below if graphic symbols or lettering is required.

Apply graphic symbols and lettering with paint-resistant, die-cut stencils, firmly secured to pavement. Mask an extended area beyond edges of each stencil to prevent paint application beyond the stencil. Apply paint so that it cannot run beneath the stencil.

PROTECTING AND CLEANING

Protect pavement markings from damage and wear during remainder of construction period.

Clean spillage and soiling from adjacent construction using cleaning agents and procedures recommended by manufacturer of affected construction.

END OF SECTION 32 17 23

Retain this notice in all Sections of Project Manual.

San Diego Unified School District Guide Specifications

Document Version: August 2021