MasterSpec 2014

SECTION 32 91 13

SOIL PREPARATION

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

GENERAL

RELATED DOCUMENTS

MANDATORY REQUIREMENT:  Retain this article in all Sections of Project Manual.

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

SUMMARY

Section includes planting soils [ and layered soil assemblies] specified by composition of the mixes.

Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

Section 31 10 00 "Site Clearing" for topsoil stripping and stockpiling.

Section 31 20 00 “Earth Moving” for bioretention basin sizing.

Section 32 92 00 "Turf and Grasses" for placing planting soil for turf and grasses.

Section 32 93 00 "Plants" for placing planting soil and mulch for plantings.

DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

DG: Decomposed aggregate surfacing materials as specified in section 32 15 40.01.

Soil Type 1: General Planting

Soil Type 2: General Turf

Paragraph below applies to both school athletic sports fields and joint use sports fields.

Soil Type 3: Sports Fields.

Uniformly Smooth Soil Surface:  A soil surface that has a consistent slope or elevation with no more than a 1/2” deviation within a 1,000 sq. ft. (10’x10’) area.  Soil surface shall be free of depressions, pits, and high spots.

UNIT PRICES

Retain this article if Work specified in this Section is measured and paid for under the provisions of unit prices. Do not include amounts. Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the unit price.

Work of this Section is affected by unit prices specified in Section 01 22 00 "Unit Prices."

BASIS OF DESIGN

The planting soil design is to be based on agricultural field testing recommendations of [on-site soils] [and] [imported soils].

When products are identified by manufacturer name with no additional notation, the use of that manufacturer name and product name is not intended to limit the selection of equal products by other manufacturers.  However, use of equal products by other manufacturers is subject to the requirements of Section 01 25 00 “Substitution Procedures.”

DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

Backfill: The earth used to replace or the act of replacing earth in an excavation. This can be amended or unamended soil as indicated.

CEC: Cation exchange capacity.

Compost: The product resulting from the controlled biological decomposition of organic material that has been sanitized through the generation of heat and stabilized to the point that it is beneficial to plant growth.

Duff Layer: A surface layer of soil, typical of forested areas, that is composed of mostly decayed leaves, twigs, and detritus.

Imported Soil: Soil that is transported to Project site for use.

Layered Soil Assembly: A designed series of planting soils, layered on each other, that together produce an environment for plant growth.

Manufactured Soil: Soil produced by blending soils, sand, stabilized organic soil amendments, and other materials to produce planting soil.

NAPT: North American Proficiency Testing Program. An SSSA program to assist soil-, plant-, and water-testing laboratories through interlaboratory sample exchanges and statistical evaluation of analytical data.

Organic Matter: The total of organic materials in soil exclusive of undecayed plant and animal tissues, their partial decomposition products, and the soil biomass; also called "humus" or "soil organic matter."

Planting Soil: On-site soil, imported soil, or manufactured soil that has been modified as specified with soil amendments and perhaps fertilizers to produce a soil mixture best for plant growth.

RCRA Metals: Hazardous metals identified by the EPA under the Resource Conservation and Recovery Act.

Subgrade: Surface or elevation of subsoil remaining after excavation is complete, or the top surface of a fill or backfill before planting soil is placed.

Subsoil: Soil beneath the level of subgrade; soil beneath the topsoil layers of a naturally occurring soil profile, typified by less than 1 percent organic matter and few soil organisms.

PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

Preinstallation Conference: Conduct conference at Project site.

If needed, insert list of conference participants not mentioned in Section 01 31 00 "Project Management and Coordination."

ACTION SUBMITTALS

Product Data: For each type of product.

Include recommendations for application and use.

Include test data substantiating that products comply with requirements.

Include sieve analyses for aggregate materials.

Bioretention Soil: Include installation instructions, substrate preparation, list of materials, application rate, testing and percolation rates.

Material Certificates and Laboratory Testing Results: Submit within 4 weeks of Notice to Proceed: For each type of [imported soil] [bioretention soil] [soil amendment and fertilizer] [and]  [organic soil amendment] before delivery to the site, according to the following:

Manufacturer's qualified testing agency's certified analysis of standard products.

Analysis of fertilizers, by a qualified testing agency, made according to AAPFCO methods for testing and labeling and according to AAPFCO's SUIP #25.

Analysis of nonstandard materials, by a qualified testing agency, made according to SSSA methods, where applicable.

Bioretention Soil

Manufacturer certification that products meet or exceed all physical properties, endurance, performance and packaging requirements.

Bioretention soils test results must be from tests    conducted within 120 days prior to delivery date to the project site.

For projects installing more than 100 cubic yards of bioretention soil, provide batch-specific test results and certifications.

Grain size analysis results of the sand component performed in accordance with ASTM D 422, Standard Test Method for Particle Size Analysis of Soils.

Grain size analysis results of the sandy loam component performed in accordance with ASTM D 422, Standard Test Method for Particle Size Analysis of Soils.

Grain size analysis results of compost component performed in accordance with ASTM D 422, Standard Test Method for Particle Size Analysis of Soils.

Agricultural soil analysis results performed in accordance with ASTM D 422, Standard Test Method for Particle Size Analysis of Soils

Organic Soil Amendments

Analysis of organic soil amendments by a qualified testing agency.

Results of organic soil amendment tests conducted within 120 days prior to delivery date to the project site.

Grain size analysis results performed in accordance with ASTM D 422, Standard Test Method for Particle Size Analysis of Soils.

Agricultural soil analysis results performed in accordance with ASTM D 422, Standard Test Method for Particle Size Analysis of Soils.

Compost: Submit the past 3 inspection reports from the local CalRecycle enforcement agency verifying compliance with specified Title14 requirements.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable.

Samples: For bioretention soil and each bulk-supplied material, 1-quart volume of each in sealed containers labeled with content, source, and date obtained. Each Sample shall be typical of the lot of material to be furnished; provide an accurate representation of composition, color, and texture.

INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in "Quality Assurance" Article.

Qualification Data: For each testing agency.

Preconstruction Test Reports: For preconstruction soil analyses specified in "Preconstruction Testing" Article.

Field quality-control reports.

QUALITY ASSURANCE

Testing Agency Qualifications: An independent, state-operated, or university-operated laboratory; experienced in soil science, soils testing, and plant nutrition; with the experience and capability to conduct the testing indicated; and that specializes in types of tests to be performed.

Laboratories: Subject to compliance with requirements, provide testing by one of the following:

Way Point Laboratory, Inc., 4741 East Hunter Ave., Suite A, Anaheim, CA 92807.

Wallace Laboratories, 365 Coral Circle, El Segundo, CA 90245.

PRECONSTRUCTION TESTING

Retain this article for preconstruction testing. Project-specific preconstruction testing of soils may be required if on-site soil or stockpiled soils were not tested before the Contract, if they are not sufficient and additional soil will be imported, or if imported soil might not comply with requirements for planting soil.

Preconstruction Testing Service: Engage a qualified testing agency to perform preconstruction soil analyses on [on-site soil] [and] [imported soil].

Notify District Construction Manager seven days in advance of the dates and times when laboratory samples will be taken.

Retain "Preconstruction Soil Analyses" Paragraph below for soil to be used or amended for landscaping purposes only if these soils are not analyzed before the Contract. Revise to suit Project and insert other characteristics required in analyses. If retaining, also retain "Preconstruction Test Reports" Paragraph in "Informational Submittals" Article.

Preconstruction Soil Analyses: For each unamended soil type, perform testing on soil samples and furnish soil analysis and a written report containing soil-amendment and fertilizer recommendations by a qualified testing agency performing the testing according to "Soil-Sampling Requirements" and "Testing Requirements" articles.

Have testing agency identify and label samples and test reports according to sample collection and labeling requirements.

SOIL-SAMPLING REQUIREMENTS

Revise this article to suit Project. Consult testing laboratory for recommendations on sampling procedures suitable for soil location, existing conditions, and required soils tests to suit Project.

General: Extract soil samples according to requirements in this article.

Retain "Sample Collection and Labeling" Paragraph below to ensure that a responsible party takes appropriate samples for representative and accurate test results; revise to suit Project.

Sample Collection and Labeling: Have samples taken and labeled by Contractor in presence of Project Inspector under the direction of the testing agency.

Revise "Number and Location of Samples" and "Procedures and Depth of Samples" subparagraphs below to require more specific requirements to suit Project.

Number and Location of Samples: Minimum of [three per acre for turf areas], and [three total for shrub areas] representative soil samples from varied locations for each soil to be used or amended for landscaping purposes.

Procedures: A representative sample shall consist of a composite of multiple sub-samples taken from the area under investigation, combined and then submitted. Discard the top inch of soil to avoid including debris that might contaminate the sample. Equipment for sampling may be a soil sampling tube, soil auger, spade, shovel, and/or trowel. Soil sampling tools shall be clean and not rusty. Do not sample if the soil is muddy, or excessively wet. Seal soil samples shall be sealed in airtight plastic bags.

Edit the next two paragraphs according to soil Types present.

Depth of Core Samples [Type 1] [and] [Type 2] Soils:

Import Soils:

At off-site stock pile – 12” to 18” depth

After placement – 6” to 8” depth

On-Site Soils:

In place at grade – 8”-10” depth

After amending and fine grading – 6” to 8” depth

[Depth of Core Samples Type 3 Soils:

Import Soils:

At off-site stockpile – 12” to 18” depth

After placement – 6” to 8” depth

On-Site Soils:

In place at grade – 8”-10” depth

Soil below DG – 8”-10” depth below bottom of DG

After amending and fine grading – 6” to 8” depth]

Labeling

On-Site Soils:  Label each sample with the date, location keyed to a site plan or other location system, visible soil condition, and sampling depth for on-site soils or source of material and suppliers name for all import soils.

Import Soils:  Label each sample with the date, source of material, and suppliers name,

Packaging and Shipping: Ship samples as directed by testing agency.

Reference Plant List:  Provide a list of plants proposed for installation as indicated on the Drawings, and a copy of these specifications, to the testing agency.

TESTING REQUIREMENTS

Revise this article to suit Project. If a different series of tests are required for different soils, copy one or more paragraphs below, designate the soil to be tested by inserts, and re-edit paragraphs for each series of tests required. Consult testing laboratory for recommendations on testing suitable for soil location, existing conditions, and intended uses of the soil to suit Project.

General: Perform tests on soil samples according to requirements in this article.

Conduct soils tests for on-site soil a minimum of two (2) times.

First Soils Test: Test in place on-site or stockpiled on-site soil to determine the condition of on-site soils.  Provide three test locations for every acre of turf and one test location for every 10,000 sq. ft of plant bed (maximum of three).  If on-site soils cannot meet the requirements of topsoil as defined herein, provide import topsoil as specified below.

Second Soils Test: If the on-site soils can be amended to meet the requirements of topsoil as specified herein, complete a second series of soils testing after the soil has been amended and leached per the recommendation of the first soils test.  If testing results are not in compliance with specifications, additional testing may be required as directed by the District Construction Manager in collaboration with the Landscape Architect, at no additional cost to the District.

Conduct soils tests for imported topsoil a minimum of three (3) times.

First Time Soils Tests:  Test stockpiled soil at the providers supply yard to determine necessary treatment to meet imported soil requirements.  Conduct a minimum of three separate soils tests to provide the most accurate analysis of stockpiled soils. A representative sample shall consist of a composite of multiple sub-samples taken from the area under investigation, combined and then submitted.

Second Time Soils Tests: Complete a second series of three (3) soils tests after the soil has been amended per the recommendation of the first soils tests.  Perform these tests prior to delivery of imported soil.  If the test results indicate additional soil amendments are necessary, continue to amend the soil and retest until the soil meets the specification requirements.

Third Time Soils Tests: Complete a third series of three (3) soils tests after the imported topsoil is placed on-site.  Complete and verify all additional soil amendment recommendations prior to planting.  If testing results are not in compliance with specifications, additional testing may be required as directed by the District Construction Manager in collaboration with the Landscape Architect, at no additional cost to the District.

Retain required "Physical Testing," "Chemical Testing," and "Fertility Testing" paragraphs below; revise and insert other tests to suit existing soil location and conditions, future uses, and other Project requirements.

Physical Testing:

Soil Texture: Soil-particle, size-distribution analysis by the following method according to SSSA's "Methods of Soil Analysis - Part 1-Physical and Mineralogical Methods":

Hydrometer Method: Report percentages of sand, silt, and clay.  Report percentage of gravel, if present.

Total Porosity: Calculate using particle density and bulk density according to SSSA's "Methods of Soil Analysis - Part 1-Physical and Mineralogical Methods."

Water Retention: According to SSSA's "Methods of Soil Analysis - Part 1-Physical and Mineralogical Methods."

Saturated Hydraulic Conductivity: According to SSSA's "Methods of Soil Analysis - Part 1-Physical and Mineralogical Methods"; at 85% compaction according to ASTM D 698 (Standard Proctor).

Chemical Testing:

CEC: Analysis by sodium saturation at pH 7 according to SSSA's "Methods of Soil Analysis - Part 3- Chemical Methods."

Clay Mineralogy: Analysis and estimated percentage of expandable clay minerals using CEC by ammonium saturation at pH 7 according to SSSA's "Methods of Soil Analysis - Part 1- Physical and Mineralogical Methods."

Generally retain "Metals Hazardous to Human Health" Subparagraph below for brownfield redevelopment sites.

Metals Hazardous to Human Health: Test for presence and quantities of RCRA metals including aluminum, arsenic, barium, copper, cadmium, chromium, cobalt, lead, lithium, and vanadium. If RCRA metals are present, include recommendations for corrective action.

Generally retain "Phytotoxicity" Subparagraph below. Some plants are more sensitive to the presence and concentrations of trace minerals.

Phytotoxicity: Test for plant-available concentrations of phytotoxic minerals including aluminum, arsenic, barium, cadmium, chlorides, chromium, cobalt, copper, lead, lithium, mercury, nickel, selenium, silver, sodium, strontium, tin, titanium, vanadium, and zinc.

Fertility Testing: Soil-fertility analysis according to standard laboratory protocol of SSSA, including the following:

Revise required items for analysis in subparagraphs below to suit Project; insert additional subparagraphs if required.

Percentage of water in saturation extract.

Electrical conductivity (ECe).

Soil reaction (acidity/alkalinity pH value).

Buffered acidity or alkalinity.

Nitrogen ppm.

Phosphorous ppm.

Potassium ppm.

Manganese ppm.

Manganese-availability ppm.

Molybdenum ppm.

Iron ppm.

Sulfur ppm.

Boron ppm.

Zinc ppm.

Zinc availability ppm.

Copper ppm.

Chloride ppm.

Sodium ppm and sodium absorption ratio (SAR).

Soluble-salts ppm.

Presence and quantities of problem materials including salts and metals cited in the Standard protocol. If such problem materials are present, provide additional recommendations for corrective action.

Other deleterious materials, including their characteristics and content of each.

Organic-Matter Content: Analysis using loss-by-ignition method according to SSSA's "Methods of Soil Analysis - Part 3- Chemical Methods."

Recommendations: Based on the test results, testing agency shall provide interpretation of data and written recommendations for soil treatments, soil amendments to be incorporated, and required leaching, to produce satisfactory planting soil suitable for healthy, viable plants as indicated on Drawings. Include, at a minimum, recommendations for nitrogen, phosphorous, and potassium fertilization, and for micronutrients.  Include results of the soil texture and percent by dry weight of soil organic matter. Include recommendations for remediation of phytotoxicity elements and deleterious materials.  Include amount and length of time to perform leaching operations, if required.

Fertilizers and Soil Amendment Rates: State recommendations in weight or volume per 1000 sq. ft. for 6-inch depth of soil and in weight or volume per cu. yd. of backfill mix.

Generally, adjust soil reaction (pH) only in soils where the pH is near the extreme ends of the acceptable range for the plants indicated.

Soil Reaction: State the recommended liming rates for raising pH or sulfur for lowering pH according to the buffered acidity or buffered alkalinity in weight or volume per 1000 sq. ft. for 6-inch depth of soil and in weight or volume per cu. yd. of backfill mix.

DELIVERY, STORAGE, AND HANDLING

Retain paragraphs in this article to suit Project. Limit inserts to only those unusual requirements not included in Section 01 60 00 "Product Requirements."

Packaged Materials: Deliver packaged materials in original, unopened containers showing weight, certified analysis, name and address of manufacturer, and compliance with state and Federal laws if applicable.

Bulk Materials:

Do not dump or store bulk materials near structures, utilities, walkways and pavements, or on existing turf areas or plants.

Provide erosion-control measures to prevent erosion or displacement of bulk materials, discharge of soil-bearing water runoff, and airborne dust reaching adjacent properties, water conveyance systems, or walkways.

Do not move or handle materials when they are wet or frozen.

Accompany each delivery of bulk fertilizers and soil amendments with appropriate certificates.

PRODUCTS

PLANTING SOILS SPECIFIED BY COMPOSITION

This article is organized around two soil sources used to produce planting soils: on-site soil and imported soil.  For each of those two sources, three Types are defined.

| ON-SITE SOIL | IMPORTED SOIL |
| --- | --- |
| Type 1 – General Planting | Type 1 – General Planting |
| Type 2 – General Turf | Type 2 – General Turf |
| Type 3 – Sports Fields | Type 3 – Sports Fields |

Retain "General" Paragraph below if soil sources were not tested before the Contract, if they are insufficient and additional soil will be imported, or if imported soil might not comply with requirements for planting soil; revise to suit Project.

General: Soil amendments, fertilizers, and rates of application specified in this article are guidelines that may need revision based on testing laboratory's recommendations after preconstruction soil analyses are performed.

Copy paragraphs below and re-edit for each type of planting-soil mix required to suit Project. Insert drawing designation. Use these designations on Drawings to identify each product.

Retain "Planting-Soil Type" Paragraph below if amending on-site soil for use as planting soil.

On-Site Planting-Soil [Type 1 – General Planting] [Type 2 – General Turf] [Type 3 –Sports Fields]: On-site soil, with the duff layer, if any, retained; modified to produce viable planting soil.

Amended Soil Composition: For bidding purposes blend unamended on-site soil with the following soil amendments and fertilizers in the following quantities to produce planting soil (actual quantities will be determined by soils test recommendations):

Revise "Ratio of Loose Compost to Soil" Subparagraph below to suit Project if compost is not fully blended into the mix but is surface applied and lightly tilled.

            - Ratio of Loose Compost to Soil: 4 cu. yd. per 1000 sq. ft. per 6 inches, or part thereof, of amended soil depth.

Weight of Soil Sulfur: 20 lbs. per 1000 sq. ft. per 6 inches, or part thereof, of amended soil depth.

Weight of Iron Sulfate: 20 lbs. 1000 sq. ft. per 6 inches, or part thereof, of amended soil depth.

Weight of Agricultural Gypsum: 100 lbs. per 1000 sq. ft. per 6 inches, or part thereof, of amended soil depth.

Weight of Superphosphate: 8 lbs. per 1000 sq. ft. per 6 inches, or part thereof, of amended soil depth.

Weight of Commercial Fertilizer: 200 lbs. per 1000 sq. ft. per 6 inches, or part thereof, of amended soil depth.

Weight of Slow-Release Fertilizer: 20 lbs. per 1000 sq. ft. per 6 inches, or part thereof, of amended soil depth.

Sand as required to increase infiltration rate to acceptable levels.

Additional Properties of on-site soil after amending and leaching:

Soil reaction of pH 6 to 8 as measured in the saturation extract (Method 21a, USDA Handbook Number 60)

Percent Organic Matter By Dry Weight

[Type 1 – General Planting areas] [and] [Type 2 - General Turf areas] Soils: 3 to 5 percent after leaching.

[Type 3 - Sports Fields Soils: 5 to 6 percent after leaching.]

Salinity: The salinity range measured in the saturation extract (Method 3a, USDA Handbook Number 60) shall be 3.0 or less dS/m. If calcium and sulfate ions both exceed 20 milliequivalents per liter in the saturation extract, the maximum salinity shall be 5.0 dS/m.

Chloride: The maximum concentration of soluble chloride in the saturation extract (Method 3a, USDA Handbook Number 60) shall be 150 mg/l (parts per million) for ornamental planting and cool season turf, and 500 mg/l (parts per million) for warm season turf.

Boron: The maximum concentration of soluble boron in the saturation extract (Method 3a, USDA Handbook Number 60) shall be 1 mg/l (parts per million).

Sodium Absorption Ratio (SAR): The maximum SAR shall be 5 measured per Method 20b, USDA Handbook Number 60.

Carbon: Nitrogen ratio shall be less than 20:1

Aluminum: Available aluminum measured with the Ammonium Bicarbonate/DTPA Extraction shall be less than 5 parts per million.

Calcium Carbonate Content: Free calcium carbonate (limestone) shall not be present.

Phytotoxic Constituent, Herbicides, Hydrocarbons: Germination and growth of monocots and dicots shall not be restricted more than 10 percent compared to the reference soil. Total petroleum hydrocarbons shall not exceed 50 mg/kg dry soil measured per the modified EPA Method No. 8015. Total aromatic volatile organic hydrocarbons (benzene, toluene, xylene and ethylbenzene) shall not exceed 0.5 mg/kg dry soil measured per EPA Methods No. 8020.

Heavy Metals: The maximum permissible elemental concentration in the soil shall not exceed the following:

Total Threshold Concentration Limit (Ammonium  Bicarbonate/DTPA Extraction TTLC Level) parts per million (mg/kilogram) dry weight basis:

Antimony: 		150 ppm.

Arsenic:		50 ppm.

Barium: 		1,000 ppm.

Beryllium:		7.5 ppm.

Cadmium: 		10 ppm.

Chromium: 		50 ppm.

Cobalt:			800 ppm

Copper:			250 ppm

Lead: 			50 ppm.

Mercury: 		2 ppm.

Molybdenum:	3,500

Nickel:			200 ppm.

Selenium:		10 ppm.

Silver: 			50 ppm.

Thallium:		70 ppm.

Vanadium:		240 ppm.

Zinc:			2,500 ppm.

If the soil pH is between 6 and 7, the maximum permissible elemental concentration shall be reduced 50 percent. No more than three metals shall be present at 50 percent or more of the above values.

Sandy Loam Gradation Limit for Type 3 Sports Fields Soil.

Gravel over 2mm: Less than 10% by weight

Sand: 75% to 85%

Sand finer than 100 mesh (0.15 mm): Less than 15%

Sand finer that 60 mesh (0.25 mm): Less than 40%

Sand larger than 32 mesh (0.5 mm): Minimum 15%

Silt: 20% maximum

Clay: 15% maximum

Silt plus Clay: Less than 35%

Total Porosity: Calculate using particle density and bulk density according to SSSA's "Methods of Soil Analysis - Part 1-Physical and Mineralogical Methods."

[Type 1 – General Planting areas] [and] [Type 2 - General Turf areas] Soils: Minimum 0.5” up to 5” per hour infiltration rate

[Type 3 - Sports Fields Soils: Minimum 2” up to 5” infiltration rate]

Unacceptable Properties: Clean soil of the following:

Unacceptable Materials: Concrete slurry, concrete layers or chunks, cement, plaster, building debris, oils, gasoline, diesel fuel, paint thinner, turpentine, tar, roofing compound, acid, and other extraneous materials that are harmful to plant growth.

Unsuitable Materials: Stones exceeding 1/2-inch in any direction, roots, plants, sod, clay lumps, and pockets of coarse sand.

Retain "Planting-Soil Type" Paragraph below if topsoil is imported, amended, and used as planting soil.  Imported and amended soil should only be used if soils test results for on-site soils indicate extremely poor soils or unacceptable toxicity levels

Imported Planting-Soil [Type 1 – General Planting] [Type 2 – General Turf] [Type 3 – Sports Fields]: Imported, naturally formed soil from off-site sources and consisting of sandy loam or loamy sand soil according to USDA textures; and modified to produce viable planting soil. Acceptable soil texture is as follows: gravel content less than 10% by weight, clay less than 15%, and clay and silt together make up less than 35%.

Type 1 – General Planting and Type 2 General Turf:

Gravel over 2mm: Less than 10% by weight.

Sand 75-85%

Silt 20% Maximum

Clay 15% Maximum

Silt plus Clay: Less than 35%

Type 3 – Sports Fields:

Gravel over 2mm: Less than 10% by weight

Sand: 75% to 85%

Sand finer than 100 mesh (0.15 mm): Less than 15%

Sand finer that 60 mesh (0.25 mm): Less than 40%

Sand larger than 32 mesh (0.5 mm): Minimum 15%

Silt: 20% maximum

Clay: 15% maximum

Silt plus Clay: Less than 35%

Sources: Take imported, unamended soil from sources that are naturally well-drained sites where topsoil occurs at least 12 inches deep, not from agricultural land, bogs, or marshes; and that do not contain undesirable organisms; disease-causing plant pathogens; or obnoxious weeds and invasive plants including, quackgrass, Johnsongrass, poison ivy, nutsedge, nimblewill, Canada thistle, bindweed, bentgrass, wild garlic, ground ivy, perennial sorrel, and bromegrass.

Amended Soil Composition: Blend imported, unamended soil based on the results and recommendations of the soil analysis to produce planting soil.

Additional Properties of Imported Soil after Amending:

Soil reaction of pH 6 to 8 as measured in the saturation extract (Method 21a, USDA Handbook Number 60).

Percent Organic Matter By Dry Weight

[Type 1 – General Planting areas] [and] [Type 2 - General Turf areas] Soils: 3 to 5 percent after leaching.

[Type 3 - Sports Fields Soils: 5 to 6 percent after leaching.]

Salinity: The salinity range measured in the saturation extract (Method 3a, USDA Handbook Number 60) shall be 3.0 or less dS/m. If calcium and sulfate ions both exceed 20 milliequivalents per liter in the saturation extract, the maximum salinity shall be 5.0 dS/m.

Chloride: The maximum concentration of soluble chloride in the saturation extract (Method 3a, USDA Handbook Number 60) shall be 500 mg/l (parts per million).

Boron: The maximum concentration of soluble boron in the saturation extract (Method 3a, USDA Handbook Number 60) shall be 1 mg/l (parts per million).

Sodium Absorption Ratio (SAR): The maximum SAR shall be 5 measured per Method 20b, USDA Handbook Number 60.

Carbon:nitrogen ratio shall be less than 20:1

Aluminum: Available aluminum measured with the Ammonium Bicarbonate/DTPA Extraction shall be less than 5 parts per million.

Calcium Carbonate Content: Free calcium carbonate (limestone) shall not be present.

Phytotoxic Constituent, Herbicides, Hydrocarbons: Germination and growth of monocots and dicots shall not be restricted more than 10 percent compared to the reference soil. Total petroleum hydrocarbons shall not exceed 50 mg/kg dry soil measured per the modified EPA Method No. 8015. Total aromatic volatile organic hydrocarbons (benzene, toluene, xylene and ethylbenzene) shall not exceed 0.5 mg/kg dry soil measured per EPA Methods No. 8020.

Heavy Metals: The maximum permissible elemental concentration in the soil shall not exceed the following:

Total Threshold Concentration Limit (Ammonium  Bicarbonate/DTPA Extraction TTLC Level) parts per million (mg/kilogram) dry weight basis:

Antimony: 		150 ppm.

Arsenic:		50 ppm.

Barium: 		1,000 ppm.

Beryllium:		7.5 ppm.

Cadmium: 		10 ppm.

Chromium: 		50 ppm.

Cobalt:			800 ppm

Copper:			250 ppm

Lead: 			50 ppm.

Mercury: 		2 ppm.

Molybdenum:	3,500

Nickel:			200 ppm.

Selenium:		10 ppm.

Silver: 			50 ppm.

Thallium:		70 ppm.

Vanadium:		240 ppm.

Zinc:			2,500 ppm.

If the soil pH is between 6 and 7, the maximum permissible elemental concentration shall be reduced 50 percent. No more than three metals shall be present at 50 percent or more of the above values.

Total Porosity: Calculate using particle density and bulk density according to SSSA's "Methods of Soil Analysis - Part 1-Physical and Mineralogical Methods."

[Type 1 – General Planting areas] [and] [Type 2 - General Turf areas] Soils: Minimum 0.5” up to 5” per hour infiltration rate

[Type 3 - Sports Fields Soils: Minimum 2” up to 5” infiltration rate]

Unacceptable Properties: Clean soil of the following:

Unacceptable Materials: Concrete slurry, concrete layers or chunks, cement, plaster, building debris, oils, gasoline, diesel fuel, paint thinner, turpentine, tar, roofing compound, acid, and other extraneous materials that are harmful to plant growth.

Unsuitable Materials: Stones exceeding 1/2-inch in any direction, roots, plants, sod, clay lumps, and pockets of coarse sand.

BIORETENTION SOIL

General: Bioretention Soil shall achieve an initial infiltration rate of at least 8-inch per hour nor more than 20 inches per hour “in situ” and a long-term, in-place infiltration rate of at least 5 inches per hour, or as directed by the District Construction Manager and Landscape Architect. Bioretention soil shall support plant growth.  Bioretention Soil shall be a mixture of fine sand, and compost, measured on a volume basis:

65% Sand

20% Sandy Loam

15% Compost

To achieve the following gradation:

85-88% Sand

8-12% Fines (from Sandy Loam Topsoil)

3-5% Organic Matter

Sand shall be free of wood, waste, coating such as clay, stone dust, carbonate, etc., or any other deleterious material. All aggregate passing the No. 200 sieve size shall be non-plastic.

Sand for Bioretention Soil shall be analyzed by an accredited lab using #200, #100, #40, #30, #16, #8, #4, and 3/8-inch sieves (ASTM D 422 or as approved by municipality), and meet the following gradation:

Sieve Size 		Percent Passing (by weight)

3/8 inch	100

No. 4	90-100

No. 8	70-100

No. 16	40-95

No. 30	15-70

No. 40	5-55

No. 100	0-15

No. 200	0-5

Note: All sands complying with ASTM C 33 for fine aggregate comply with the above gradation requirements.

Sandy loam for Bioretention Soil shall be free of wood, waste, coating such as stone dust, carbonate, etc., or any other deleterious material. All aggregate passing the No. 200 sieve size shall be non-plastic.

Sandy loam soil should comply with the following specifications on USDA soil textural classification scheme by weight:

50-74% sand

0-48% silt

2-15% clay

Note: all sands complying with ASTM C 33 for fine aggregate comply with the above gradation requirements.

Compost for Bioretention Soil shall be a well decomposed, stable, weed free organic matter source derived from waste materials including yard debris, wood wastes or other organic materials.  Compost shall have a dark brown color and a soil like odor. Compost exhibiting a sour or putrid smell, containing recognizable grass or leaves, or is hot (120F) upon delivery or rewetting is not acceptable.  Compost shall be produced at a facility inspected and regulated by the Local Enforcement Agency for CalRecycle.  Comply with Title14 requirements of the Process to Further Reduce Pathogens (PFRP), Fecal coliform and Salmonella testing and pathogen and EPA, 40 CFR 503 regulations.

| Property | Method | Requirement |
| --- | --- | --- |
| pH, Units | Saturation Paste | 6 to 8.5 |
| EC, dS/m | Saturation Extract | 0 to 10 |
| Boron, ppm | Saturation Extract | less than 2.5 |
| Moisture content, % | Gravimetric | 30 to 60 |
| Bulk Density, lbs/cubic yard |  | 500 to 1100 |
| Organic Matter, % of Dry Weight | Loss on Ignition | 35% to 75% |
| Carbon to Nitrogen Ratio |  | 15:1 to 20:1 |
| Maturity | Solvita | 5 or above |
| Stability | Solvita | 5 or above |
| Particle Size | Sieve Analysis |  |
| Pass 1/2 inch sieve |  | 90%-100% |
| Pass #200 sieve |  | max 5% |
| CA Title 22 Metals | Title 22 |  |
| Antimony (Sb) |  | Less than 150 ppm |
| Arsenic (As) |  | Less than 50 ppm |
| Barium (Ba) |  | Less than 1,000 ppm |
| Beryllium (Be) |  | Less than 7.5 ppm |
| Cadmium (Cd) |  | Less than 10 ppm |
| Chromium (Cr) |  | Less than 50 ppm |
| Cobalt (Co) |  | Less than 800 ppm |
| Copper (Cu) |  | Less than 250 ppm |
| Lead (Pb) |  | Less than 50 ppm |
| Mercury (Hg) |  | Less than 2 ppm |
| Molybdenum (Mo) |  | Less than 3,500 ppm |
| Nickel (Ni) |  | Less than 200 ppm |
| Selenium (Se) |  | Less than 10 ppm |
| Silver (AG) |  | Less than 50 ppm |
| Thallium (TI) |  | Less than 70 ppm |
| Vanadium (V) |  | Less than 240 ppm |
| Zinc (Zn) |  | Less than 2,500 ppm |
| Pathogen |  |  |
| Salmonella | Title 14 | < 3 MPN per 4 gms |
| Fecal Coliform |  | <1000 MPN per 1 gm |
| Physical contaminants |  |  |
| Plastic Metal and Glass, %> 4mm | % by Weight | < 1 |
| Sharps, % > 4mm | % by Weight | 0 |

Bioretention Soil shall be free of roots, clods stones larger than 1-inch in the greatest dimension, pockets of coarse sand, noxious weeds, sticks, lumber, brush and other litter.  It shall not be infested with nematodes, or undesirable disease-causing organisms such as insects and plant pathogens.  Bioretention soil mix shall be friable and have sufficient structure in order to give good tilth and aeration to the soil.

Gradation limits – The definition of the soil should be the following USDA classification scheme by weight:

Sand  85-92%

Silt  14% maximum

Clay  5% maximum

Permeability Rate - Hydraulic conductivity rate shall be not less the 8-inch per hour nor more than 20 inches per hour when tested in accordance with USDA Handbook Number 60, method 34b or other approved methods.

Analysis for pH, salinity and nutrient levels shall be submitted for approval prior to acceptance.  Nutrient tests should include the testing laboratory recommendations for supplemental additions to the soil as calculated by the amount of material to be added per volume of soil for the type of plants to be grown in the soil.

| Property | Method | Requirement |
| --- | --- | --- |
| pH, Units | Saturation Paste | 6.0 to 8.0 |
| EC, dS/m | Saturation Extract | 0.5 to 2.5 |
| Boron, ppm | Saturation Extract | less than 2.5 |
| Chloride, ppm | Saturation Extract | less than 500 |
| Sodium Adsorption Ratio |  | less than 5.0 |
| Carbon to Nitrogen Ratio |  | 10:1 to 20:1 |
| Organic Matter, % of Dry Weight | Loss on Ignition | 2 to 5 |
| Extractable Nutrients, dry weight basis | Ammonium Bicarbonate/DPTA Extraction |  |
| phosphorus, ppm |  | 10 to 40 |
| potassium, ppm |  | 100 to 200 |
| iron, ppm |  | 24 to 35 |
| manganese, ppm |  | 0.6 to 6 |
| zinc, ppm |  | 1 to 8 |
| copper, ppm |  | 0.3 to 5 |
| magnesium, ppm |  | 50 to 150 |
| sodium, ppm |  | 0 to 100 |
| sulfur, ppm |  | 25 to 500 |
| molybdenum, ppm |  | 0.1 to 2 |
| aluminum, ppm |  | less than 3.0 |

Bioretention Soil shall be analyzed by an accredited lab using #200, 1/4-inch, 1/2-inch, and 1-inch sieves (ASTM D 422 or as approved by municipality), and meet the following gradation:

Sieve Size 			Percent Passing (by weight)

1-inch 						99-100

1/2-inch 					90-100

1/4-inch 					40-90

No. 200 					Less than 5%

INORGANIC SOIL AMENDMENTS

Insert percentages of carbonates, calcium, and magnesium in "Lime" Paragraph below if required.

Lime: ASTM C 602, agricultural liming material containing a minimum of 80 percent calcium carbonate equivalent and as follows:

Retain one of two "Class" subparagraphs below. Class T is more finely ground and quicker acting but dustier than Class O.

Class: T, with a minimum of 99 percent passing through a No. 8 sieve and a minimum of 75 percent passing through a No. 60 sieve.

Class: O, with a minimum of 95 percent passing through a No. 8 sieve and a minimum of 55 percent passing through a No. 60 sieve.

Form: Provide lime in form of ground calcitic limestone or ground mollusk shells.

Retain one of or both "Sulfur" and "Iron Sulfate" paragraphs below if required. Do not use aluminum sulfate. Revise descriptions and insert proprietary products if required.

Sulfur: Granular, biodegradable, and containing a minimum of 90 percent elemental sulfur, with a minimum of 99 percent passing through a No. 6 sieve and a maximum of 10 percent passing through a No. 40 sieve.

Iron Sulfate: Granulated ferrous sulfate containing a minimum of 20 percent iron and 10 percent sulfur, and conforming to requirements of the California Food and Agricultural Code.

Perlite: Horticultural perlite, soil amendment grade.

If using ground-up, recycled gypsum board debris as a soil amendment, replace or revise "Agricultural Gypsum" Paragraph below or insert another paragraph for this additional material. Coordinate with planting-soil mixes retained in "Planting Soils Specified by Composition" Article.

Agricultural Gypsum: Minimum 90 percent calcium sulfate, finely ground with 90 percent passing through a No. 50 sieve.

Sand: Clean, washed, natural or manufactured, free of toxic materials.

Sand finer than 100 mesh (0.15 mm): Less than 15%

Sand finer that 60 mesh (0.25 mm): Less than 40%

Sand larger than 32 mesh (0.5 mm): Minimum 15%

ORGANIC SOIL AMENDMENTS

Compost is a widely used bulk organic soil amendment and a recycled product. Because it is applied at much greater rates than fertilizer, compost has a significant cumulative effect on nutrient availability and may reduce or eliminate initial fertilization. Consider each plant's pH and soluble-salt requirements and how they relate to the compost being used and the resulting soil-compost mix.

Compost: Well decomposed, stable, and weed-free organic matter produced by composting feedstock, and bearing USCC's "Seal of Testing Assurance," and as follows:

Compost shall be produced at a facility inspected and regulated by the Local Enforcement Agency for CalRecycle.

Comply with Title14 requirements of the Process to Further Reduce Pathogens (PFRP), Fecal coliform and Salmonella testing and pathogen and EPA, 40 CFR 503 regulations

Feedstock limited to leaves and plant parts only; includes straw and alfalfa.

Mushroom, manures, peat mosses, green waste and food waste composts are acceptable.

Composted wood products are conditionally acceptable (stable humus must be present). Wood products based on redwood or cedar are not acceptable. Wood derivatives, including sawdust and chipped construction waste are not acceptable.

Sewage sludge and animal waste are not acceptable.

Compost shall have a dark brown color and a soil like odor.

Compost exhibiting a sour or putrid smell, containing recognizable grass or leaves, or is hot (120F) upon delivery or rewetting is not acceptable.

Comply with Title14 requirements of the Process to Further Reduce Pathogens (PFRP), Fecal coliform and Salmonella testing and pathogen and EPA, 40 CFR 503 regulations.

Particle Size: The maximum particle size shall be 0.5-inch

Calcium carbonate shall not be present if to be applied on alkaline soils.

The salt content shall be less than 6 millimho/cm @ 25° C. (ECe less than 6) in a saturated paste extract.

Additional Properties:

| Property | Method | Requirement |
| --- | --- | --- |
| pH, Units | Saturation Paste | 5.5 to 8 |
| Soluble-Salt Concentration |  | Less than 5 dS/m |
| SAR |  | Less than 5 |
| Chloride | Method 3a, USDA | Less than 1000 mg/l |
| EC, dS/m | Saturation Extract | 0 to 10 |
| Boron, ppm | Saturation Extract | less than 1 |
| Silicone Content (Acid insoluble ash) |  | Less than 50% |
| Moisture content by weight, % | Gravimetric | 35-55% |
| Bulk Density, lbs/cubic yard |  | 500 to 1100 |
| Organic Matter, % of Dry Weight | Loss on Ignition | 50-65% |
| Carbon to Nitrogen Ratio |  | Less than 20:1 |
| Maturity | Solvita | 5 or above |
| Stability | Solvita | 5 or above |
| Particle Size | Sieve Analysis |  |
| Pass 3/4 inch sieve |  | 98% |
| Pass 1/2-inch sieve |  | 90%-100% |
| Pass #4 screen |  | 80% or more |
| CA Title 22 Metals | Title 22 |  |
| Antimony (Sb) |  | Less than 150 ppm |
| Arsenic (As) |  | Less than 50 ppm |
| Barium (Ba) |  | Less than 1,000 ppm |
| Beryllium (Be) |  | Less than 7.5 ppm |
| Cadmium (Cd) |  | Less than 10 ppm |
| Chromium (Cr) |  | Less than 50 ppm |
| Cobalt (Co) |  | Less than 800 ppm |
| Copper (Cu) |  | Less than 250 ppm |
| Lead (Pb) |  | Less than 50 ppm |
| Mercury (Hg) |  | Less than 2 ppm |
| Molybdenum (Mo) |  | Less than 3,500 ppm |
| Nickel (Ni) |  | Less than 200 ppm |
| Selenium (Se) |  | Less than 10 ppm |
| Silver (AG) |  | Less than 50 ppm |
| Thallium (TI) |  | Less than 70 ppm |
| Vanadium (V) |  | Less than 240 ppm |
| Zinc (Zn) |  | Less than 2,500 ppm |
| Pathogen |  |  |
| Salmonella | Title 14 | < 3 MPN per 4 gms |
| Fecal Coliform |  | <1000 MPN per 1 gm |
| Physical contaminants |  |  |
| Plastic Metal and Glass, %> 4mm | % by Weight | < 1 |
| Sharps, % > 4mm | % by Weight | 0 |

Retain "Sphagnum Peat" Paragraph below if required; sphagnum peat is an acidic peat.

Sphagnum Peat: Partially decomposed sphagnum peat moss, finely divided or of granular texture with 100 percent passing through a 1/2-inch sieve, a pH of 3.4 to 4.8, and a soluble-salt content measured by electrical conductivity of maximum 5 dS/m.

FERTILIZERS

Retain one or more of "Superphosphate," "Commercial Fertilizer," and "Slow-Release Fertilizer" paragraphs below. Revise descriptions, insert additional types, and name proprietary products if required.

Superphosphate: Commercial, phosphate mixture, water soluble; a minimum of 20 percent available phosphoric acid.

Commercial Fertilizer: Commercial-grade complete fertilizer of neutral character, consisting of fast- and slow-release nitrogen, minimum 50 percent derived from natural organic sources of urea formaldehyde, phosphorous, and potassium in the following composition:

Composition: Nitrogen, phosphorous, and potassium in amounts recommended in soil reports from a qualified testing agency.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

Gro-Power.

Wilbur-Ellis Agribusiness.

Kellogg Garden Products.

Or Equal.

Slow-Release Fertilizer: Granular or pelleted fertilizer consisting of 50 percent water-insoluble nitrogen, phosphorus, and potassium in the following composition:

Composition: Nitrogen, phosphorous, and potassium in amounts recommended in soil reports from a qualified testing agency.

Manufacturers: Subject to compliance with requirements, provide products by one of the following:

Gro-Power.

Wilbur-Ellis Agribusiness.

Kellogg Garden Products.

Or Equal.

Retain "Chelated Iron" Paragraph below if required; indicate location(s) of use and application rate(s) elsewhere in Part 2 or Part 3.

Chelated Iron: Commercial-grade FeEDDHA for dicots and woody plants, and commercial-grade FeDTPA for ornamental grasses and monocots.

EXECUTION

GENERAL

Place planting soil and fertilizers according to requirements in this section.

Verify that no foreign or deleterious material or liquid such as paint, paint washout, concrete slurry, concrete layers or chunks, cement, plaster, oils, gasoline, diesel fuel, paint thinner, turpentine, tar, roofing compound, or acid has been deposited in planting soil.

Proceed with placement only after unsatisfactory conditions have been corrected.

PLACING AND MIXING TYPE 1 – GENERAL PLANTING SOIL AND TYPE 2 – GENERAL TURF PLANTING SOIL OVER EXPOSED SUBGRADE

Generally retain this article if on-site soil or imported soil will be amended and applied to areas where on-site soils have been excavated and removed from the site or that have poor on-site soils regarded as subgrade.

General: Apply and mix [unamended on-site soil] [import topsoil] with amendments on-site to produce required planting soil. Do not apply materials or till if on-site soil or subgrade is frozen, muddy, or excessively wet.

Subgrade Preparation: Cross rip subgrade to a minimum depth of 3 inches. Remove stones, sticks, roots, rubbish, and other extraneous matter 1-inch and larger in any dimension, and legally dispose of off District's property.

Retain subparagraph below to knit planting soil to subsoil and to improve soil drainage. Retain for caliche or hardpan subgrades, poorly drained subgrades, or subgrades that heavily compact during construction.

Apply, add soil amendments, and mix with machine rototill approximately half the thickness of unamended soil over prepared, loosened subgrade according to "Mixing" Paragraph below. Mix with machine rototill thoroughly into top 4 inches of subgrade.  Machine rototill a minimum of two alternating passes, crossing at 90 degree angles.  Spread remainder of planting soil.

Delete paragraph below if TYPE 1 – GENERAL PLANTING SOIL is chosen above

Mixing: Spread unamended soil to total depth of 12 inches, but not less than required to meet finish grades after mixing machine rototill with amendments and natural settlement. Do not spread if soil or subgrade is muddy or excessively wet.

Retain option in "Amendments" Subparagraph below if applying compost component of planting-soil mix to surface and lightly tilling in lieu of blending compost throughout planting-soil mix.

Amendments: Apply soil amendments, [compost,] and fertilizer, if required, evenly on surface, and thoroughly blend them with unamended soil by machine rototill to produce planting soil. Machine rototill a minimum of two alternating passes, crossing at 90 degree angles.

Mix fertilizer with planting soil no more than seven days before planting.

Lifts: Apply and mix unamended soil and amendments in lifts not exceeding 6 inches in loose depth for material compacted by compaction equipment, and not more than [4 inches] [6 inches] in loose depth for material compacted by hand-operated tampers.

Compaction: Compact each blended lift of planting soil to 80 percent minimum to 85 percent maximum of maximum Standard Proctor density according to ASTM D 698 [except where a different compaction value is indicated on Drawings].

Leaching: Leach amended soils with a minimum of 6” of water over a 2 week period or as determined by the soils test recommendations to reduce SAR, ECE, and Sodium to acceptable levels as determined by the soils test recommendations.

BLENDING TYPE 1 – GENERAL PLANTING SOIL AND TYPE 2 – GENERAL TURF PLANTING SOIL IN PLACE

Generally retain this article if unamended, on-site soil is amended without excavating to produce planting soil.

General: Mix amendments with in-place, unamended on-site soil to produce required planting soil. Do not deep rip, apply materials, or rototill if on-site soil or subgrade is frozen, muddy, or excessively wet.

Deep Ripping: Deep rip and loosen planting areas to a depth of 12 inches in two directions, crossing at 90 degree angles. Access roads used during construction activities within planting areas shall be deep ripped and loosened to a depth of 3 feet in two opposite directions, crossing at 90 degree angles.

Preparation: Machine Rototill unamended on-site soil in planting areas to a minimum depth of 6 inches. Machine rototill a minimum of two alternating passes, crossing at 90 degree angles. Remove stones, sticks, roots, rubbish, and other extraneous matter 1 inch and larger in any dimension, and legally dispose of them off District's property.

Retain option in "Mixing" Paragraph below if applying compost component of planting-soil mix to surface and lightly tilling in lieu of blending compost throughout planting-soil mix.

Mixing: Apply soil amendments[, compost,] and fertilizer, if required, evenly on surface, and thoroughly blend them into full depth of unamended, in-place soil with machine rototill. Rototill a minimum of two alternating passes to a minimum depth of 6 inches, crossing at 90 degree angles to produce planting soil.

Mix fertilizer with planting soil no more than seven days before planting.

Compaction: Compact blended planting soil to 75 to 80 percent of maximum Standard Proctor density according to ASTM D 698 [except where a different compaction value is indicated on Drawings].

Leaching: Leach amended soils with a minimum of 6” of water over a 2 week period or as determined by the soils test recommendations to reduce SAR, ECE, and Sodium to acceptable levels as determined by the soils test recommendations.

      - BLENDING TYPE 3 – SPORTS FIELD IMPORT SOIL

Generally retain this article if project includes a sports field and on-site soil or approved imported soil will be amended and applied to areas where on-site soils have been excavated and removed from the site or that have poor on-site soils regarded as subgrade.

General: Do not deep rip, apply materials, or rototill if on-site soil or subgrade is frozen, muddy, or excessively wet.

        - Excavate on-site soil to a depth of 15” below finish grade.  Scarify subgrade soil in a cross pattern to a depth of 3” in two directions. Remove stones, sticks, roots, rubbish, and other extraneous matter 1 inch and larger in any dimension and legally dispose of off District's property.

# Retain paragraph below if onsite decomposed granite is used for the 5” section of subsoil. Delete if native screened material is used for the 5” section of subsoil.

        - Place existing or imported decomposed granite with an infiltration rate of at least 2” per hour to a depth of not more than 5”.  Place approved import topsoil to a depth of not less than 10” to finish grade on top of decomposed granite.

# Retain paragraph below if onsite native soil is used for the 5” section of subsoil. Delete if decomposed granite is used for the 5” section of subsoil.

        - Screen excavated site soil to remove all rocks and debris 1/2” and greater in size.  Place screened site soil with an infiltration rate of at least 2” per hour to a depth of not more than 5”.  Place approved on-site or import topsoil to a depth of not less than 10” to finish grade on top of decomposed granite.

# Retain paragraph below if on-site native sub-soil has an infiltration rate of less than 1” per hour.

        - Install 4” perforated PVC drainage system at low end(s) of field per specification section 33 46 00.

        - Compaction: Compact blended planting soil to 80 percent minimum to 85 percent maximum of maximum Standard Proctor density according to ASTM D 698 [except where a different compaction value is indicated on Drawings].

        - Leaching: Leach amended soils with a minimum of 6” of water over a 2 week period or as determined by the soils test recommendations to reduce SAR, ECE, and Sodium to acceptable levels as determined by the soils test recommendations.

PLACING BIORETENTION SOIL

Imported backfill material for the bioretention zones should be placed in a relatively loose condition, no rolling or other heavy equipment, to promote the planned infiltration of water, through the bioretention soil mix layer.

Bioretention soil shall be installed in six (6) to twelve (12) inch lifts and thoroughly watered to provide settlement and natural compaction.  No mechanical compaction is allowed. After natural compaction has been completed, add, if needed, additional bioretention soil to proposed finish grade as indicated on the plans.

Rake bioretention soil as needed to level out.

Vehicular traffic, construction equipment shall not drive-on, move onto, or disturb the bioretention soil once placed and water compacted.

Erosion and Sediment Control practices during construction shall be employed to protect the long-term functionality of the bioretention basin/swale.  The following practices shall be followed for this reason:

1.	Provide erosion control in the contributing drainage areas to the facility and stabilize upslope areas.

2.	Do not use facilities as sediment control facilities, unless installation of all bioretention-related materials are withheld towards the end of construction allowing the temporary use of the location as a sediment control facility, and appropriate excavation of sediment is provided prior to installation of bioretention materials.

Place a three inch layer of mulch on the surface of the bioretention soil unless otherwise indicated on the Drawings.

FINISH GRADING

Finish Grading: Grade planting soil to a smooth, uniform surface plane with loose, uniformly fine texture. Roll and rake to remove ridges, and fill depressions to meet finish grades. Rake out and remove all rocks and material 1/2-inch size and larger.  Prior to approval, remove ridges greater than 1/2” and fill depressions greater than 1/2” within a 1,000 sq. ft. (10’x10’) area.

Retain first paragraph below if finish grades are shown on Drawings.

Finish grades shall be as indicated on the Drawings.

Finish grades shall be measured at the top surface of surface materials.

Molding and rounding of the grades shall be provided at all changes of slope.

Take every precaution to protect and avoid damage to new and existing sprinkler heads, irrigation lines, and other underground utilities during grading and conditioning operations.

Finish grades shall be acceptable to Project Inspector before planting operations begin.

Retain first two paragraphs below if finish grades are not indicated on Drawings.

Finish grade shall ensure positive drainage of the site with all surface drainage away from buildings, other structures, and walls with flow towards storm drains and catch basins.

Planting surfaces shall be graded with no less than 2 percent surface slope for positive drainage.

Sports Fields turf areas shall be graded with a 1% minimum and 1.5% maximum surface slope for positive drainage unless indicated otherwise on the drawings.

FIELD QUALITY CONTROL

Testing Agency: Engage a qualified testing agency to perform the following tests and inspections.

Compaction: Test planting-soil compaction after placing each lift and at completion using a densitometer or soil-compaction meter calibrated to a reference test value based on laboratory testing according to ASTM D 698. Space tests at no less than one for each [1000 sq. ft.] [2000 sq. ft.] <Insert dimension> of in-place soil or part thereof.

<Insert name of test>: <Insert requirement>.

<Insert name of inspection>: <Insert requirement>.

Soil will be considered defective if it does not pass tests[ and inspections].

Prepare test[ and inspection] reports.

Label each sample and test report with the date, location keyed to a site plan or other location system, visible conditions when and where sample was taken, and sampling depth.

PROTECTION

Revise "Protection Zone" Paragraph below to suit Project. Delete if referenced Section is not used.

Protection Zone: Identify protection zones according to Section 01 56 39 "Temporary Tree and Plant Protection."

Protect areas of in-place soil from additional compaction, disturbance, and contamination. Prohibit the following practices within these areas except as required to perform planting operations:

Revise subparagraphs below to suit Project.

Storage of construction materials, debris, or excavated material.

Parking vehicles or equipment.

Vehicle traffic.

Foot traffic.

Erection of sheds or structures.

Impoundment of water.

Excavation or other digging unless otherwise indicated.

If planting soil or subgrade is overcompacted, disturbed, or contaminated by foreign or deleterious materials or liquids, remove the planting soil and contamination; restore the subgrade as directed by Landscape Architect and replace contaminated planting soil with new planting soil.

CLEANING

Protect areas adjacent to planting-soil preparation and placement areas from contamination. Keep adjacent paving and construction clean and work area in an orderly condition.

Remove surplus soil and waste material including excess subsoil, unsuitable materials, trash, and debris and legally dispose of them off District's property unless otherwise indicated.

END OF SECTION 32 91 13

Retain this notice in all Sections of Project Manual.

San Diego Unified School District Guide Specifications

Document Version: August 2021